# Compliance Software Test Requirements

This chapter contains the procedures used to test and certify vendor’s compliance software as acceptable for compliance with Title 24 Part 6.  Compliance software must also follow all modeling rules specified in Chapter 5.  The tests used to verify software functionality and accuracy of simulation results are referred to as the Reference Method. The tests fall into the following categories:

- Tests to verify that the software is evaluating thermal loads and the response of the HVAC systems to these loads in a manner that is acceptable. These tests reference ASHRAE Standard 140-2007, Standard Method of Test for Evaluation of Building Energy Analysis Computer Programs.

- Tests that verify that compliance software is capable of modeling envelope, lighting, HVAC and water heating efficiency features and provides precise estimates of energy tradeoffs and reasonably accurate predictions of building energy consumption.

- Tests to verify that the Standard Design (baseline building) is created correctly, e.g. that the baseline HVAC system is properly specified, that other components of the baseline are correctly defined and that rules that fix and restrict inputs (such as schedules and plug loads) are properly applied. These tests do not verify simulation outputs, but may require simulations to be run to specify inputs that are dependent on system sizing.

- The Reference Method is designed to cover representative software functionality for building envelope, space uses, lighting, daylighting, HVAC and water heating, both for simulation performance and for proper implementation of ACM rules specified in Chapter 5.  The California Energy Commission reserves the right to add Ruleset Implementation Tests or Software Sensitivity Tests to verify existing or future compliance software requirements.  Moreover, the California Energy Commission reserves the right to adjust the passing criteria for the Software Sensitivity Tests to reflect the capabilities of commonly available energy simulation programs.

## General Requirements

### Scope

The Compliance Software must satisfy the requirements contained in this section.

The Compliance Software shall be capable of modeling at least 50 thermal zones.

The Compliance Software shall be capable of modeling at least 15 separate HVAC systems.

### Calculation Methods

The Compliance Software shall calculate the annual consumption of all end uses in buildings, including fuel and electricity for:

- HVAC (heating, cooling, fans, and ventilation);

- Lighting (both interior and exterior);

- Receptacles and miscellaneous electric;

- Service water heating;

- Process energy uses;

- Commercial refrigeration systems; and

- All other energy end uses that typically pass through the building meter.

The Compliance Software shall perform a simulation on an hourly time interval (at a minimum) over a one year period (8760 hours) with the ability to model changes in weather parameters, schedules, and other parameters for each hour of the year. This is achieved by specifying a 24-hour schedule for each day of the week plus holidays.

#### Calculating Design Loads

The software shall be capable of performing design load calculations for determining required HVAC equipment capacities and air and water flow rates using accepted industry calculation methods for the standard design.

#### Checking Simulation Output for Unmet Loads

The software shall be capable of checking the output of the energy analysis module for the proposed design to ensure that space conditions are maintained within the tolerances specified (maximum of 300 unmet load hours per year).

#### Adjusting Capacities

For the baseline building, the software shall be capable of modifying capacities, temperatures or flow rates for baseline building HVAC system components causing excessive unmet loads hours according to the procedures in Chapter 2.

#### Error Handling

The software shall identify error conditions when unmet loads exceed 300 hours, prevent completion of the compliance analysis, and provide information to the user describing the error that has occurred and what steps the user should take to remedy the situation.

### Climate Data

The Compliance Software shall perform simulations using the official California Energy Commission weather files and design conditions documented in Joint Appendix 2.

The Compliance Software shall calculate solar radiation on exterior surfaces on an hourly basis from the values of direct normal irradiance and diffuse horizontal irradiance contained in the climate data, taking ground reflectance into account.

### Utility Rates

The Rating Software shall be capable of simulating time-of-use rates and apply both demand and energy charges for each time period of the rate schedule.

### Time Dependent Valued (TDV) Energy

The Compliance Software shall be capable of applying the Energy Commission TDV multipliers for each hour of the simulation. See California Joint Appendix 3.

### Thermal Mass

The calculation procedures used in the Compliance Software shall account for the effect of thermal mass on: loads due to occupants, lights, solar radiation, and transmission through building envelope; amount of heating and cooling required to maintain the specified space temperature schedules; and variation in space temperature.

### Modeling Space Temperature

The Compliance Software shall incorporate a dynamic simulation of space temperature which accounts for:

- Dynamics in change in heating and cooling setpoint temperatures;

- Deadband between heating and cooling thermostat settings;

- Temperature drift in transition to setback or setup thermostat schedules;

- Temperature drift in periods when heating or cooling capability are scheduled off;

- Temperature drift when heating or cooling capability of the system is limited by heating or cooling capacity, air flow rate, or scheduled supply air temperature; and

- Indirectly conditioned thermal zones, where the temperature is determined by internal loads, heat transfer through building envelope, and heat transfer between thermal zones.

### Heat Transfer between Thermal zones

The Compliance Software shall be capable of modeling heat transfer between a thermal zone and adjacent thermal zones.

The Compliance Software shall account for the effect of this heat transfer on the space temperature, space conditioning loads, and resulting energy use in the thermal zone and in the adjacent thermal zones.

### Control and Operating Schedules

The Compliance Software shall be capable of modeling control and operating schedules which can vary by:

- The hour of the day;

- The day of the week; and

- Holidays treated as a special day of the week.

The Compliance Software shall be capable of explicitly modeling all of the schedules specified in Appendix 5.4B of this manual.

### Loads Calculation

The loads calculations described in this section relate to the simulation engine and not to the procedure used the design engineer to size and select equipment.

#### Internal Loads

The Compliance Software shall be capable of calculating the hourly cooling loads due to occupants, lights, receptacles, and process loads.

The calculation of internal loads shall account for the dynamic effects of thermal mass.

The Compliance Software shall be capable of simulating schedules for internal loads in the form given in Appendix 5.4B.

The simulation of cooling load due to lights shall account for:

- The effect of the proportion radiant and convective heat, which depends on the type of light, on the dynamic response characteristic; and

- A portion of heat from lights going directly to return air, the amount depending on the type and location of fixture.

#### Building Envelope Loads

The Compliance Software shall calculate heat transfer through walls, roofs and floors for each thermal zone, accounting for the dynamic response due to thermal characteristics of the particular construction as defined in the Building Descriptors in Chapter 5.

The calculation of heat transfer through walls and roofs shall account for the effect of solar radiation absorbed on the exterior surface, which depends on orientation and absorptance of the surface.

The Compliance Software shall calculate heat transfer through windows and skylights, accounting for both temperature difference and transmission of solar radiation through the glazing.

Calculation of cooling load due to transmission of solar radiation through windows and skylights shall account for:

- The angular incidence of the direct beam sunlight and the angular and spectral dependence of the solar properties.

- The variation of thermal properties of the fenestration system with ambient temperature.

- Orientation (azimuth and tilt of surface).

- The effect of shading from overhangs side fins, louvers or neighboring buildings or terrain.

#### Infiltration

The Compliance Software shall be capable of simulating infiltration that varies by the time of day and day of the week. Schedules are provided in Appendix 5.4B.

#### Natural Ventilation

The Compliance Software shall be capable of simulating natural ventilation.

### Systems Simulation

#### General

The Compliance Software shall be capable of modeling:

- The baseline building systems defined in Chapter 5,

- The lighting, water heating, HVAC and miscellaneous equipment detailed in Chapter 5

- All compulsory and required features as detailed in Chapter 5

The capability to model multiple zone systems shall allow at least 15 thermal zones to be served by one multiple zone system.

The Compliance Software shall be capable of modeling plenum air return.

#### HVAC Zone Level Systems

The Compliance Software shall be capable of simulating the effect on space temperature and energy use of:

- Limited capacity of terminal heating devices;

- Limited capacity of terminal cooling devices; and

- Limited rate of air flow to thermal zones.

#### HVAC Secondary Systems and Equipment

The Compliance Software shall be capable of simulating the effect on energy use and space temperature in thermal zones served by the HVAC system of:

- Limited heating capacity of an HVAC system; and

- Limited cooling capacity of an HVAC system.

The simulation of HVAC systems shall account for:

- Temperature rise of supply air due to heat from supply fan, depending on the location of the fan;

- Temperature rise of return air due to heat from return fan;

- Temperature rise of return air due to heat from lights to return air stream; and

- Fan power as a function of supply air flow in variable air volume systems.

#### HVAC Primary Systems and Equipment

The Compliance Software shall be capable of simulating the effect on energy use of limited heating or cooling capacity of the central plant system.

If the Compliance Software is not capable of simulating the effect of limited heating or cooling capacity of the central plant system on space temperature in thermal zones dependent on the central plant system for heating and cooling, then it shall issue a warning message when loads on the central plant system are not met.

#### Equipment Performance Curves

The Compliance Software shall be capable of modeling the part load efficiency and variation in capacity of equipment as follows:

- Furnaces as a function of part load;

- Boilers as a function of part load, supply hot water temperature, and return hot water temperature;

- Water-cooled compressors including heat pumps and chillers as a function of part load, evaporator fluid, or air temperature and condensing fluid temperature;

- Air-cooled compressors including heat pumps, direct expansion cooling and chillers as a function of part load, ambient dry-bulb temperature, and wet-bulb temperature returning to the cooling coil;

- Evaporative cooling systems as a function of ambient wet-bulb temperature; and

- Cooling towers as a function of range and ambient wet-bulb temperature.

#### Economizer Control

The Compliance Software shall be capable of modeling integrated air- and water-side economizers.

#### Heat-Recovery Water Heating

The Compliance Software shall be capable of modeling heat recovery water heating from the following sources:

- Double bundled chiller;

- Refrigerant desuperheater as part of a packaged HVAC unit;

- Heat exchanger on the condenser water loop; and

- Heat-recovery water-to-water heat pumps operating off of the condenser or chilled water loop.

## Special Documentation and Reporting Requirements

### Building Envelope

#### Roof Radiative Properties

The user shall enter three-year aged roof reflectance and emittance for roofs that have been certified by the Cool Roof Rating Council.

#### Ground Reflectance

When specific design features are incorporated to increase or reduce reflectance for the purpose of enhancing daylighting, there can be variation between the proposed design and the standard design, when appropriately documented. In cases where the default value is overridden, the user is required to submit documentation identifying the test procedure that was used to establish the non-default values.

#### Exterior Wall Surface Properties

The default value for emittance is 0.90. The default value for reflectance is 0.30. There is no default for roughness. The default values may be overridden only in cases when the lower reflectance can be documented by manufacturers’ literature or tests.

### Interior Lighting

#### Regulated Interior Lighting Power

Complete lighting plans and space plans are required for the tailored method. Prescriptive compliance forms for the tailored method shall be developed and these shall be verified by the plans examiner.

Whenever any of the special allowance exceptions (footnotes in Standards Table 140.6-C) are claimed, the compliance software shall make an entry on the compliance forms to indicate that verification is required.

With the tailored lighting method, the software shall make an entry in the special features section on the California compliance forms that the tailored lighting method has been used to determine the lighting power for this space and that all necessary prescriptive tailored lighting forms and worksheets documenting the lighting and its justification shall be provided as part of the compliance documentation and be approved independently.

#### Indoor Lighting Power (see 5.4.4)

Compliance software shall print all applicable lighting forms and report the lighting energy use and the lighting level (Watts/ft2) for the entire project. Compliance software shall report “No Lighting Installed” for nonresidential spaces with no installed lighting.  Compliance software shall report “Default Residential Lighting” for residential units of high rise residential buildings and hotel/motel guest rooms.

If the modeled Lighting Power Density (LPD) is different than the actual LPD calculated from the fixture schedule for the building, Compliance software shall model the larger of the two values for sizing the mechanical systems and for the compliance run. Compliance software shall report the larger value on PERF-1.  Lighting levels schedules shall be adjusted by any lighting Control Credit Watts, if input by the user.

Lighting power is not modeled in unconditioned spaces that are modeled, but lighting in those spaces is required to meet the prescriptive requirements for regulated unconditioned spaces such as commercial and industrial storage spaces and parking garages.  When these types of spaces are entered the compliance software must report in the Special Features section that these spaces must comply with the prescriptive requirements for such spaces.

#### Complete Building Method

The Complete Building method triggers special reporting requirements. The compliance software shall report the following on the compliance form:

“The Complete Building method requires that at least 90% of the building contain conditioned space, and that at least 90% of the building conform to the building type classification. This condition must be verified by the building official.”

#### Design Illumination Setpoint

Spaces that have low design illuminance levels, below the ranges specified in Appendix 5.4A, shall provide documentation that show the design illuminance to be used as the daylight illumination setpoint.

#### Primary and Secondary Daylighted Area

The default primary daylight area for sidelighting is a band near the window with a depth equal to the distance from the floor to the top of the window. The default secondary daylight area for sidelighting is a band beyond the primary daylighted area that extends a distance double the distance from the floor to the top of the window. Other daylight areas may be defined with appropriate documentation.

#### Light Heat Gain Distribution

Values within the ranges of Table 8 may be used when following the rules in the 2009 ASHRAE HOF. Other values may be used when manufacturers’ literature and/or testing data is available, and adequate documentation is provided to the California Energy Commission.

### Exterior Lighting

If the lighted façade area exceeds exterior wall area or if door linear footage exceeds 25% of building perimeter, the software shall produce a warning.

### Infiltration Data

When credit is taken for reductions in infiltration (see above), test results or research reports shall be provided that document and support the inputs used for the baseline building and the proposed design.

### HVAC Exceptional Conditions

#### Equipment Sizing

When the any proposed equipment size for secondary equipment or central plant equipment does not match the equipment size listed on construction documents, an exceptional condition shall be reported on compliance forms.

#### Custom Occupant, Fan, Heating or Cooling Schedules

Custom hourly schedules may be used with appropriate documentation from project schedules. The default schedules from Appendix 5.4B apply unless they are overridden. The user must ensure that the HVAC schedule operating hours serve all of the occupied spaces, and spaces should not be combined when their weekly occupancy varies by more than 40 full load equivalent operating hours (FLEOH).

#### Equipment Performance Curves

(Is documentation required if non-default HVAC equipment performance curves are used?  For example, COOL-CAP-FT, COOL-EIR-FT, etc.)

(Note documentation required if credits for fault detection and diagnostics and/or airflow testing are taken.)

For equipment with a capacity less than or equal to 135,000 Btu/h, but larger than 65,000 Btu/h, the user may not enter data on the temperature dependent equipment performance. However, the compliance software vendor may work with manufacturers to collect such data and build this data into the compliance software. The user may either select equipment for which the compliance software vendor has collected or use the defaults.

#### Natural Ventilation

The air flow rate for the proposed design shall be determined using sound engineering methods and supporting documentation shall be provided.

#### Condenser Flow Type

The default is fixed condenser flow for water-cooled condensers. If the variable-flow is selected, the software must indicate that supporting documentation is required on the output forms.

#### Evaporative Cooling Performance Curves

User may input curves or use default curves. If defaults are overridden, the software must indicate that supporting documentation is required on the output forms.

#### Condenser Heat Recovery Effectiveness

As designed. The software must indicate that supporting documentation is required on the output forms if heat recovery is specified.

#### Boiler Heat Loss

Default is 2% for electric boilers and heat-exchangers and 0% for fuel-fired boilers. If the user overrides the default, supporting documentation is required.

#### Boiler Minimum Unloading Ratio

If the user does not use the default value the software must indicate that supporting documentation is required on the output forms.

#### Chiller Minimum Unloading Ratio

. If the user does not employ the default values, supporting documentation is required.

#### Cooling Tower Setpoint Control

If the user does not use the default control, supporting documentation is required.

#### Exterior Lighting Control

Documentation should be provided for lighting controls other than standard and evidence should be provided to support the reduction in lighting power (PAF).

## ASHRAE Standard 140-2007 Tests

This method of testing is provided for analyzing and diagnosing building energy simulation software using software-to-software and software-to-quasi-analytical-solution comparisons. The methodology allows different building energy simulation programs, representing different degrees of modeling complexity, to be tested by comparing the predictions from other building energy programs to the simulation results provided by the Compliance Software in question.

Compliance software must pass ASHRAE 140-2007 tests, but these tests are not part of the Reference Method.

## Ruleset Implementation Tests

### Introduction

The Building Energy Sensitivity Tests in the Reference Method suite of tests verifies that the simulation engine produces results that are within an acceptable range. The tests in this section are intended to verify that the software correctly constructs the Standard Design model and applies rules of the 2013 Nonresidential ACM appropriately to the proposed and Standard Design models. The rule set implementation tests cover representative portions of the rules for building envelope, lighting, daylighting, space use data and HVAC.  For each test there is a set of three models defined:

- User Model – the user model contains the user inputs for the as-designed building. In most cases, the values for the Proposed Design will be taken from user inputs with no modification. However, there are some cases where the building input is prescribed for the Proposed Design, or constrained by mandatory minimums or other rules.

- Proposed Design Model – the proposed model is defined by the rules in the ACM Reference Manual and created by the vendor software and is the building modeled for compliance. This model takes user inputs for building geometry, building envelope, lighting and HVAC and is used in the compliance simulation.

- Standard Design Model – this is the baseline model defined by the ACM Reference Manual modeling rules, and is the basis for comparison that determines whether a building passes compliance using the performance method.

These tests do not require that simulation outputs be verified, but they do require that simulation input files for the Proposed Design and Standard Design are properly constructed according to the rules in the ACM Reference Manual. Some tests require that sizing runs be performed, for HVAC inputs whose values are dependent on autosized Standard Design systems.

### Overview

The test runs described in this section represent the Title-24 2013 Non-Residential ACM code compliance calculation and use the following Prototype Models- Small Office Building, Medium Office Building, Large Office Building, Warehouse Building and Small Hotel. For further details on the Prototype Models, refer to Section III of the Software Compliance Specification document. Each of the Standard Design test cases shall be created by modifying the Prototype Model as described in Section IV of this document. The modified Prototype Model shall form the proposed case for each test run. The Standard Design model shall be generated by compliance software as per the rules in the 2013 Nonresidential ACM. The Standard Design and proposed model files for each of test cases shall then be evaluated to verify that:

- The Standard Design building envelope constructions are correctly substituted for exterior opaque surfaces and fenestrations.

- Fenestration area in the Standard Design building is reduced in accordance with the 2013 Nonresidential ACM Manual, when the Proposed Design fenestration area is greater than 40% of the exterior wall.

- Skylight area in the Standard Design building is adjusted in accordance with the 2013 Nonresidential ACM Manual, when applicable.

- Default schedules of operation are applied for both the Standard Design building and the Proposed Design.

- The proposed and Standard Design cases use the same defaults or tailored inputs for internal loads as required by the 2013 Nonresidential ACM Manual.

- The Standard Design building lighting system is correctly specified and that exterior lighting is modeled.

- Receptacle loads, process loads is modeled according to the rules in the 2013 Nonresidential ACM Manual.

- The Standard Design building use the correct system types as prescribed in Table 5 of the 2013 Nonresidential ACM Manual.

- An economizer (of the right type) is included in the Standard Design building if required.

- The primary and secondary Standard Design building systems are properly specified and sized.

- Fan brake horsepower is correctly specified for the Standard Design building.

- Prescribed modeling assumptions are applied for both the Standard Design building and the Proposed Design.

- Overhangs are modeled in the Proposed Design for Test Case 7 but not the Standard Design building.

- Unconditioned spaces are modeled.

- Other Standard Design building specifications and/or modeling assumptions are correctly applied.

As the software developer verifies the various test conditions, the input and output files should be annotated with comments or other methods to demonstrate that the modeling rules specified in the 2013 Nonresidential ACM Manual are correctly applied. Software developers should use the output format spreadsheets, included in Appendix B, to report the results of these tests. These annotated files shall then be submitted to the Commission for further evaluation. Any errors discovered shall be corrected by making modifications to the software; the runs shall be repeated; and the new results shall be annotated for submittal to the Commission.

The Standard Design Tests are labeled using the format:

Standard Design Fenestration Test/Medium Office -CZ-15, Exterior Envelope Run 01

Run name number

Climate Zone

Prototype Model Type

Test Type

### Ruleset Implementation Tests

The following tests shall be performed to verify that the compliance software correctly creates the Standard Design model and applies modeling rules as per the requirements of the Nonresidential ACM 2013. The proposed model shall be created by modifying the appropriate Prototype Model type. The Standard Design Model for each test case shall be generated automatically by the complaince software.

The intent of each test run, characteristics of the proposed model and inputs to be verified in the Standard Design model are described below.

- Standard Design Exterior Envelope Test/Small Office-CZ-6, Run01– This test will verify that the applicant software models the  wall, floor and  roof construction correctly in the Standard Design model and that the envelope and fenestration performance requirements for the Standard Design are modelled as per the requirements of 2013 Nonresidential ACM.

The user model is a small office building in climate zone 6 with the following envelope characteristics:

- Low sloped concrete roof with assembly U-value of  0.065, Aged Solar Reflectance of 0.75 and Thermal emittance of 0.78

- Wood-framed wall with assembly U-value of 0.095

- Mass floor with asssembly U-value of 0.052

- Fixed window with U-factor of 0.25, SHGC of 0.20 and VT of 0.45

The following inputs in the Proposed Design and Standard Design model file shall be verified and reported in the output form provided in Appendix 3C:

- Standard Design roof, wall and floor construction assembly type

- Standard Design roof, wall and floor overall U-value

- Standard Design roof, wall and floor construction assembly layer inputs

- Standard Design roof aged solar reflectance and thermal emittance

- Window U-factor, SHGC and VT

- Standard Design Exterior Envelope Test/Small Office-CZ-15, Run02 – This test will verify that the applicant software models the wall, floor, roof construction and fenestrations correctly and that the envelope performance requirements for the Standard Design are modelled as per the 2013 Nonresidential ACM.

The user model is a small office building in climate zone 15 with the following envelope characteristics:

- Steep sloped metal building roof with assembly U-factor of  0.055, Aged Solar Reflectance of 0.60 and Thermal emittance of 0.70

- Metal-framed wall with assembly U-factor of 0.056

- Slab on grade floor with floor F-factor of 0.70

- Horizontal overhang of 2 feet on south facing windows and vertical fins of 2 feet on right and left of west facing windows

The following inputs in the Proposed Design and Standard Design model file shall be verified and reported in the output form:

- Standard Design roof, wall and floor construction assembly type

- Standard Design roof, wall and floor construction assembly layer inputs

- Standard Design roof, wall overall U-value

- Standard Design floor slab F-factor

- Standard Design roof aged solar reflectance and thermal emittance.

- Window overhangs for south and west windows

- Standard Design Exterior Envelope Test/Small Hotel-CZ-15, Run03 – This test will verify that the applicant software models the  wall, floor and  roof construction correctly in the Standard Design model and that the envelope and fenestration performance requirements for the Standard Design are modelled as per the requirements of 2013 Nonresidential ACM.

The proposed model is a small hotel building in climate zone 15 with the following characteristics:

- Low sloped Metal building Roof with assembly U-value of  0.055, Aged Solar Reflectance of 0.60 and Thermal emittance of 0.70

- Metal-framed wall with assembly U-value of 0.8

- Mass Floor with asssembly U-value of 0.052

- Fixed windows in the first floor with U-factor of 0.25, RSHGC of 0.20 and V.T of 0.47

- Operable windows in guest room windows with U-factor of 0.42, SHGC of 0.18 and V.T of 0.35

The following inputs in the Standard Design model file shall be verified and reported in the output form:

- Standard Design roof, wall and floor construction assembly type

- Standard Design roof, wall and floor construction assembly layer inputs

- Standard Design roof, wall and floor U-value

- Standard Design roof aged solar reflectance and thermal emittance.

- Window U-value, SHGC and VT

- Standard Design Exterior Envelope Test/Medium Office-CZ6, Run04 – This test will verify that mandatory minimum opaque envelope insulation requirements are applied.  The user model is a small office building in climate zone 6, with a metal-framed wall containing R-5 continuous insulation on the exterior and a U-factor of 0.138.

  - For this test, the proposed model should be undefined, and the compliance simulation should not run.

- Standard Design Fenestration Test/Large Office-CZ-6, WWR Run05 - Run 05 tests whether the applicant software determines the window area of the Standard Design model as per the rules in 2013 Nonresidential ACM.

The user model is the Large office building in climate zone 6 with an overall window-to-wall ratio (WWR) of 52% made of a continuous band of glass distributed evenly across all facades. For this run, the building is rotated 15 degrees to the east in azimuth.

The following inputs in the Proposed Design and Standard Design model file shall be verified and reported in the output form:

- WWR

- Standard Design Fenestration Test/Large Office-CZ-6, WWR Run06 - Run 06 tests whether the applicant software determines the window area of the Standard Design model as per the rules in 2013 Nonresidential ACM.

The user model is the Large office building in climate zone 6 with overall building WWR-46% and 50% WWR in the west, 40% in South, 45% in East and 50% in the North façade.

The following inputs in the Proposed Design and Standard Design model file shall be verified and reported in the output form:

- Window Area (and WWR) for each orientation

- Window Area (and Overall WWR) for the building

- Standard Design Skylight Test/Warehouse-CZ-6, Run07 – This tests verifies whether the applicant software models the Standard Design skylight as per the performance requirements for skylights in 2013 Nonresidential ACM. This test checks whether the applicant software determines the skylight area of the Standard Design model correctly.

The user model is a Warehouse building in climate zone 6 with an overall skylight-to-roof ratio (SRR) of 7%. Curb mounted glass Skylight  with U-value 0.55, SHGC 0.20  and VT 0.40. The building has ceiling heights of 10 feet and a LPD of 0.8 W/ft2.

The following inputs in the Standard Design model file shall be verified and reported in the output form:

- Skylight U-factor, SHGC and VT, and SRR

- Standard Design Skylight Test/Warehouse-CZ-6, SRR Run08  – This test checks whether the applicant software determines the skylight area of the Standard Design model correctly.

The proposed model is a Warehouse building in climate zone 6 with ceiling heights of 20 feet and a lighting power density of 0.8 W/ft2. The SRR of the proposed building is 15% and 80% of the building area gets daylighting from skylights or sidelights.

The following Standard Design inputs shall be verified:

- Standard Design SRR, Standard Design total daylit area as a fraction of gross building area

- Standard Design Skylight Test/Warehouse-CZ-6, SRR Run09  – This test checks whether the applicant software determines the skylight area of the Standard Design model correctly.

The proposed model is a Warehouse building in climate zone 6 with ceiling heights of 20 feet and a lighting power density of 0.8 W/ft2. The SRR of the proposed building is 5% and 55% of the building area gets daylighting from skylights or sidelights.

The following Standard Design inputs shall be verified:

- Standard Design SRR, Standard Design total daylit area as a fraction of gross building area

- Standard Design Occupancy Test/Medium Office-CZ-6, Whole Building Method Run10  – This test verifies whether the applicant software inserts the correct Standard Design inputs for schedules, occupant density, equipment power density, lighting power density, hot water load  and  ventilation rates using the Complete Building lighting method and in accordance with 2013 Nonresidential ACM. The user model is an office occupancy with:

- Occupant Density- 50 ft2/person

- Equipment Power Density- 3 W/ft2

- Lighting Power Density- 1.2 W/ft2

- Hot Water Load- 106 Btu/person

- Ventilation Rate- 0.15 cfm/ft2

The Proposed Design and Standard Design Standard Design model inputs shall be verified for:

- Schedules (verified shall include occupancy schedule, lighting schedule, receptacle schedule, hot water schedule, infiltration schedule and elevator schedule)

- Occupant Density

- Equipment Power Density

- Lighting Power Density

- Hot Water Load

- Ventilation Rate

- Standard Design Occupancy Test/Large Office- CZ-6, Area Category Run11 – This test verifies whether the applicant software inserts the correct Proposed Design and Standard Design inputs for schedules, occupant density, equipment power density, lighting power density, hot water load  and  ventilation rates in accordance with 2013 Nonresidential ACM. This run tests the capability of the applicant software to model Standard Design inputs for multiple space types using the Area Category lighting method.

The user model is the Large office building in climate zone six with the following characteristics:

- First Floor- Retail space occupancy- Occupant Density 33 ft2/person, EPD-2W/ft2, LPD-1W/ft2, Ventilation rate- 0.75 cfm/ft2, Hot Water Load- 120 Btuh/person

- Second Floor- Medical occupancy- Occupant Density 100 ft2/person, EPD-1.5 W/ft2, LPD-1W/ft2, Ventilation rate- 0.15 cfm/ft2, Hot Water Load- 160 Btuh/person

- Third to 12th Floor North, South, West and East zones -Offices occupancy- Occupant Density 150 ft2/person, EPD-1.5 W/ft2, LPD-0.75 W/ft2, Ventilation rate- 0.15 cfm/ft2, Hot Water Load- 120 Btuh/person

- Third and Fifth Floor Core zones- Lobby occupancy- Occupant Density 100 ft2/person, EPD-0.5 W/ft2, LPD-1.5 W/ft2, Ventilation rate- 0.15 cfm/ft2

- Fourth, Seventh, 10th and 12th Floor Core Zones- Conference occupancy- Occupant Density 100 ft2/person, EPD-1.0 W/ft2, LPD-1.2 W/ft2, Ventilation rate- 1 cfm/ft2, Hot Water Load- 60 Btuh/person

- Sixth, Eighth Floor Core Zone- Restroom occupancy- Occupant Density 100 ft2/person, EPD-0.2 W/ft2, LPD-0.5 W/ft2, Ventilation rate- 0.15 cfm/ft2

- Ninth and 11th Floor Core Zones- Corridor occupancy- Occupant Density 100 ft2/person, EPD-0.2 W/ft2, LPD-0.5 W/ft2, Ventilation rate- 0.15 cfm/ft2

For each space type, the Proposed Design and Standard Design model inputs shall be verified for:

- Schedules (verified shall include occupancy schedule, lighting schedule, receptacle schedule, hot water schedule, infiltration schedule and elevator schedule)

- Occupant Density

- Equipment Power Density

- Lighting Power Density

- Hot Water Load

- Ventilation Rate

- Standard Design Tailored Lighting Method/Medium Office-CZ-6, Run12 – This test verifies if the applicant software applies the requirements of the Tailored Lighting Method appropriately.

The User Model is a medium office building in climate Zone 6 with the following chracteristics:

- Perimeter zones on all floors are modelled as main entry spaces  using the Tailored Lighting Method. The space has a general lighting of 0.5 W/ft2

- Core zones  on all floors are modelled as waiting spaces using the Tailored Lighting Method. The space has a general lighting of 0.75 W/ft2  and decorative lighting of 0.2 W/ft2

For each space type, the Proposed Design and Standard Design model inputs shall be verified for:

- Lighting Power Density for main entry area

- Lighting Power Density General Lighting Power for waiting area

- Lighting Power Density Custom Lighting Power for waiting area

- Lighting Schedules

- Standard Design Tailored Lighting Method/Medium Office-CZ-6, Run13  – This test verifies if the applicant software applies the requirements of the Tailored Lighting Method appropriately.

Proposed Model is a medium office building in climate Zone 6 with the following chracteristics:

- Perimeter zones on all floors are modelled as main entry spaces  using the Tailored Lighting Method. The space has a general lighting of 0.8 W/ft2 and task lighting of 0.1 W/ft2

- Core zones  on all floors are modelled as waiting spaces  using the Tailored Lighting Method. The space has a general lighting of 0.6 W/ft2, floor display lighting of 0.1 W/ft2, and decorative lighting of 1.2 W/ft2

For each space type, the Proposed Design and Standard Design model inputs shall be verified for:

- Lighting Power Density for main entry area

- Lighting Power Density General Lighting Power for waiting area

- Lighting Power Density Custom Lighting Power for waiting area

- Lighting Schedules

- Standard Design Lighting Control Test/Small Office-CZ-6, PAF Run14  – This test verifies if the applicant software models lighting power density and schedules for eligible Califonia Power adjustment factors as required by the 2013 Nonresidential ACM.

The proposed model is a small office building (area=5500 sq ft) with demand responsive lighting controls. The model files shall be examined to verify whether adjustment factors are applied as per the requirements in the 2013 Nonresidential ACM.

For this test, the lighting power density of the proposed model and Standard Design are verified.

- Standard Design Lighting Control Test/Warehouse-CZ-6, Daylightcontrol Run15  – This test verifies the ability of the applicant software to model daylighting controls as per the requirements of the 2013 Nonresidential ACM.

The user model is a warehouse building in CZ-6 with the following characteristics:

- The building has three spaces – Office, Fine Storage and Bulk Storage. The office space gets daylighting from four windows. Fine Storage gets daylighting from skylights and bulk storage gets both sidelighting from windows and skylighting.

- Daylighting controls are installed in primary and skylit daylit areas in all spaces and in the secondary daylit area of the Bulk Storage.

- The installed general lighting power controlled by daylight controls in the office space is 150 Watts.  The installed lighting power in the primary daylit zone is 150 Watts and the installed lighting power in the secondary daylit zone is 150 Watts.

- The illuminance setpoint for the office spaces is set at 400 lux and the illuminance setpoint in the fine storage and bulk storage are each set at 250 lux.

- The reference position for the office space is 5 feet from the exterior wall, at the midpoint between the two interior walls, for the primary daylit zone, and 12 feet from the exterior wall, at the midpoint between the two interior walls, for the secondary daylit zone

The Proposed Design and Standard Design model inputs shall be verified for:

For the office space primary daylit zone and secondary daylit zone:

- Lighting Power in Daylit Zone – for this input, the lighting power for general lighting is assumed to be uniform throughout the space for the Standard Design, so the fraction of lighting power in the space in the primary daylit zone and secondary daylit zone is equal to the fraction of space floor area that is in the primary daylit zone

- Secondary Daylit Zone Adjustment Factor – by space type, from Appendix 5.4A

- Daylight control type

- Reference position and illuminance setpoint

For the bulk storage space:

- Lighting Power in the Daylit Zone

- Daylight control type

- Reference position and illuminance setpoint

- For the fine storage space:

- Lighting Power in the Daylit Zone

- Daylight control type

- Reference position and illuminance setpoint

- Standard Design Lighting Control Test/Retail-CZ-6, Daylightcontrol Run16  – This test verifies the ability of the applicant software to model daylighting controls as per the requirements of the 2013 Nonresidential ACM.

The user model is a retail building in CZ-6 with the following characteristics:

- The building has five spaces – Front Entry, Point of Sale, Front Retail, Core Retail and Back Space. The front retail space has daylighting from 5 feet high glass on the exterior with a sill height of 3.74 ft.  This test only tests the daylighting control in the front retail space.

- Daylighting controls are installed in primary and skylit daylit areas in all spaces and in the secondary daylit area of the Bulk Storage.

- The front retail space has a general lighting power of 1.6 W/ft2 and task and floor display accent lighting in the space of 0.75 W/ft2 in the front half of the space closest to the exterior window.

- The illuminance setpoint is set at 950 lux in all spaces.

- The reference position for the front retail space is 5 feet from the exterior wall, at the midpoint between the two interior walls, for the primary daylit zone, and 12 feet from the exterior wall, at the midpoint between the two interior walls, for the secondary daylit zone.

The Proposed Design and Standard Design model inputs shall be verified for:

For the front retail space, primary daylit zone and secondary daylit zone:

- Lighting Power Density, General Lighting – this is calculated from the room cavity ratio and illuminance level

- Lighting Power in Daylit Zone – this input defines the lighting power that is controlled by daylight dimming controls

- Secondary Daylit Zone Adjustment Factor – by space type, from Appendix 5.4A

- Daylight control type

- Reference position and illuminance setpoint

- Standard Design Ext. Light, SHW Test/Large Office-CZ-6, Run17  – This test verifies if the 2013 Nonresidential ACM rules for exterior lighting are applied accurately for exterior lighting power, exterior lighting control and schedules.  This test also verifies if the Service Hot Water systems are modelled correctly.

The user model is a large office building in climate zone 6, with the following characteristics:

  - Hot water load of 106 Btu/h-person

  - Thermal efficiency of 0.78

  - An exterior hardscape comprising of:

    - Driveway of dimensions 20x30 feet with lighting load of 1115 Watts

    - Parking lot  of dimensions 180x56 feet with lighting load of 6566 Watts

    - 2 sidewalks  of dimensions  6x30 feet  with total lighting load of 977 Watts

    - A building entrance door light of 90 Watts.

    - 50 square feet of hardscape ornamental light of 2 Watts

    - Lighting load of 6566 Watts on the building façade.

    - Signage of 40 square feet with  lighting load of 80 Watts

This test verifies that the Proposed Design and standard design are specified correctly for:

  - Service Water Heating:  thermal efficiency, hot water load

  - Exterior Lighting Power

- Standard Design HVAC Test/Small Office-CZ-6, System Type Run18  – This test  checks  if the applicant software models the Standard Design HVAC system in accordance with the requirements of the 2013 Nonresidential ACM.

The proposed model is a small office building in climate zone 6 with the following characteristics:

- DX cooling

- Cooling COP-3.84

- Gas Furnace Heating

- Thermal Efficiency – 80%

- Constant Volume Fan

The following Standard Design model inputs shall be verified:

- HVAC System Type

- Heating and Cooling Type

- Heating and Cooling Efficiency

- System Sizing

- Maximum and minimum supply air temperature

- Fan Control Method

- Cooling and heating efficiency curve

- Cooling, Heating and Fan Schedule

- Standard Design HVAC Test/Medium Office-CZ-6, System Type Run19  – This test  checks  if the applicant software models the Standard Design HVAC system in accordance with the requirements of the 2013 Nonresidential ACM.

The proposed model is a medium office building in climate zone 6 with the following characteristics:

- Core, Mid and Top Bottom Zones are computer rooms with a zone Cooling load= 110,000 Btuh-Packaged Single Zone unit and constant volume fan

- All Perimeter zones- DX Cooling and Hot water Boiler with reheat  and variable volume fan

- Cooling Efficiency- COP=4 for packaged single zone units; COP=3.8 for VAV units serving perimeter zones

- Heating Efficiency – 82% for VAV unit serving perimeter zones

The following Standard Design model inputs shall be verified:

- HVAC System Type

- Heating and Cooling Type

- Heating and Cooling Efficiency

- System Sizing

- Maximum and minimum supply air temperature

- SAT Reset Control

- Economizer Type and limits

- Fan Power

- Fan Control Method

- Cooling and heating capacity adjustment curve

- Cooling and heating efficiency curve

- Cooling, Heating and Fan Schedule

- Terminal heat type and capacity

- Terminal minimum stop

- Terminal heat control type

- Boiler type and Number of boilers

- Boiler Heat loss, boiler minimum unloading ratio

- Hot water supply and return temperature

- Standard Design HVAC Test/Large Office-CZ-6, System Type Run20  – This test  checks  if the applicant software models the Standard Design HVAC system in accordance with the requirements of the 2013 Nonresidential ACM.

The proposed model is a Large office building in climate zone 6 with the following characteristics:

- Basement Zone is a computer room with cooling only dedicated packaged DX system.

- All other  zones have a Built-up VAV system.

- Cooling Efficiency- COP 6.2 for chiller; EER 11, SEER 14 for packaged DX system

- Heating Efficiency – 82% for boiler; not applicable for packaged DX system

The following Standard Design model inputs shall be verified:

- HVAC System Type

- Heating and Cooling Type

- Heating and Cooling Efficiency

- System Sizing

- Sizing Factor (zone,  system)

- Maximum and minimum supply air temperature

- Sat Reset Control

- Economizer Type and limits

- Fan Power

- Fan Control Method

- Cooling, Heating and Fan Schedule

- Terminal heat type and capacity

- Boiler type and Number of boilers

- Boiler Heat loss, boiler minimum unloading ratio

- Boiler Performance Curve

- Hot water supply and return temperature

- Boiler pump type

- Pump motor power and efficiency

- Pump part load curve.

- Chiller Type and Number of chillers

- Chiller fuel, capacity and efficiency

- Chiller Minimum Unloading ratio

- Chiller cooling capacity and adjustment curves

- Chilled water supply and return air temperature

- Condenser Type

- Cooling Tower Fan control type and horse power

- Cooling Tower Set Point Control

- Pump Control Type, Motor Power, Efficiency and design flow rate

- Standard Design HVAC Test/Warehouse-CZ-6, System Type Run21  – This test checks if the applicant software models the Standard Design HVAC system in accordance with the requirements of the 2013 Nonresidential ACM.

The proposed model is a Warehouse building in climate zone 6 with the following characteristics:

- Zones are served by a gas furnace heat only system.

- Heating Efficiency – 84%

The following Standard Design model inputs shall be verified-

- HVAC System Type

- Heating and Cooling Type

- Heating and Cooling Efficiency

- System Sizing

- Maximum and minimum supply air temperature

- Sat Reset Control

- Economizer Type and limits

- Fan Power

- Fan Control Method

- Cooling, Heating and Fan Schedule

- Standard Design HVAC Test/Hotel-CZ-6, System Type Run22   – This test  checks  if the applicant software models the Standard Design HVAC system in accordance with the requirements of the 2013 Nonresidential ACM.

The proposed model is a ten-story Hotel in climate zone 6 with a full-service restaurant, and an assembly area that uses demand control ventilation.

- Guestrooms are served by a four-pipe fan coil with water-cooled chiller and boiler

- Building has a water-side economizer

The following Standard Design inputs shall be verified:

- System Type is four-pipe fan coil for the hotel, and kitchen is served by a dedicated kitchen system with exhaust meeting ACM requirements

- Assembly area contains demand control ventilation in Standard Design

- Standard Design building does not contain a water-side economizer

- Standard Design HVAC Test/MediumOffice-CZ-6, System Type Run23  – This test  checks  the Standard Design building for an existing, altered building that has the roof replaced with a metal building roof, R-15 continuous insulation, and that has the windows replaced on the South façade with low-e, double glazed windows with U=0.40, SHGC=0.33 and VT=0.50.  The windows on the North, East and West facades are existing, unchanged single-paned windows with U=0.55, SHGC=0.56, VT=0.6.

### Results Comparison

The applicant shall perform all tests specified in Section IV and report the outputs in the forms provided in Appendix 3C. Note that the Standard Design for some inputs, such as cooling efficiency and pump power, are dependent upon the autosizing of the HVAC equipment. The ruleset implementation tests do not check that the autosized capacity matches the Reference Method, but rather, that the Standard Design input is properly defined in relation to the autosized capacity.

## Software Sensitivity Tests

This section details the eligibility requirements for an applicant simulation program for use as compliance software under the 2013 Title 24 ACM. A series of quantitative tests called the Software Sensitivity Tests shall be performed and results compared against predetermined Reference results to demonstrate that the applicant software is acceptable for use in code compliance. There are a total of 106 tests. All the test cases described here shall be performed and results summarized in the forms contained in Appendix 3B.

#### Overview

The applicant software shall perform a suite of Software Sensitivity Tests to demonstrate that their performance is acceptable for code compliance.  The applicant software test results shall be compared against a base case called the Reference tests. The Reference Test case is the corresponding match of a particular test case simulated already on EnergyPlus (v.7.2) engine. The Reference Model results, as determined by the Commission, are tabulated in Appendix 3B.

The test cases will assess the applicant software’s sensitivity to various inputs ranging from envelope thermal conductance to HVAC system performance. Each case tests the impact of the input component on building end use energy and annual TDV. The following six building components will be tested through a series of tests:

- Opaque envelope

- Glazing

- Lighting

- Daylighting

- Receptacle loads

- HVAC System Parameters

#### Prototype Models

The Software Sensitivity Tests are performed on four Prototype Models which are a subset of the DOE Prototype Building Models developed by PNNL for analysis of ASHRAE Standard 90.1. The Prototype Models are EnergyPlus model input files of the DOE prototype building models, modified to comply with the requirements of Title-24 2013. The Prototype Models will be the reference baseline runs for the test cases. The applicant software shall replicate the four building models using the same inputs as the Prototype Models. The models so replicated will be the applicant baseline models for the test cases.

A summary of the Prototype Models is provided in Appendix 3A. Detailed EnergyPlus input files of the Reference baseline models are available from the California Energy Commission’s Building Energy Efficiency Software Consortium webpage http://bees.archenergy.com/

Prototype Models used for Software Sensitivity test cases are:

- Medium Office Building:

The Medium Office Building model is a three storied rectangular building with an overall area of 53600 square feet. It has a window to wall ratio of 33% with fenestration distributed evenly across all four facades. The zones are served by DX cooling and Gas furnace heating with hot water reheat. For more details refer Appendix 3A.

- Large Office Building:

The large office building has twelve floors and a basement floor with glass windows with a window to wall ratio of 40% on the above-grade walls. The total area of the building is 498,600 square feet.  The HVAC system type used VAV system. For more details refer Appendix 3A.

- Stand-Alone Retail:

The stand-alone retail is a single story rectangular building measuring 178 ft by 139 ft. The total area is 24695 square feet. Windows are located only on the street facing façade and occupy 25.4% of that facade. The building is divided into five thermal zones which are served by various system types as described in Appendix 3A.

- Strip Mall Building:

The strip mall building area of 22,500 square feet. It has ten zones each with rooftop units. The building has windows in the street facing façade and has an overall window to wall ratio of 10.5%. For more details refer to Appendix 3A.

#### Climate Zones

The Software Sensitivity Test cases use building models for five of the 16 California climate zones. Most tests are performed with two or three climate zones to capture the sensitivity of the input characteristics to extremes in weather conditions. The test cases are performed on climate zones 6, 15 and 16, which represent mild, hot and cold climates respectively. Daylighting tests are performed on climate zone 7, which has a high annual daylighting potential and climate zone 1, representative of the climate with least annual daylighting potential.

Table 1-Climate Zones Tested

| Climate Zone | Example City |
| --- | --- |
| 1 | Arcata |
| 6 | Torrence |
| 7 | San Diego Lindbergh |
| 15 | Palm Springs-Intl |
| 16 | Blue Canyon |

#### Labeling Test Runs

Each test case in the Software Sensitivity test is labeled uniquely to make it easier to keep track of the runs and to facilitate analysis. The following scheme is used:

01CZ15StandAloneRetail-EnvelopeRoofInsulation

Input Parameter

Prototype Model

Climate Zone

Test Number

#### Test Criteria

Applicant software vendors shall perform a series of computer runs. Each of these runs shall be a systematic variation of the applicant base case model as described in 3.4.1.7. The applicant test case results will be compared to the reference results to verify that applicant software meets the requirements of the ACM. Simulation results for each test case will be compiled in forms provided in Appendix 3B. Compiled results will include annual site energy consumption for each end-use, overall site energy consumption, total unmet load hours and annual TDV and % variation of annual TDV and total end use site energy.

The annual TDV % variation shall be calculated using the formula:

TDV%= (TDVb – TDVn)/TDVb

Where, TDV% is the TDV % variation,

TDVn is the annual TDV for test case number n and

TDVb is the annual TDV for the base case run.

In order to be accepted, the applicant software should fulfill the passing criteria as determined by the Commission:

For each test case,

The change in energy for the test case must be in the same direction as the Reference Method test case result, and must be equal to the Reference Method test case percentage change in TDV energy, plus or minus 0.5% of baseline TDV energy.

If any of the tests required for the Title 24 compliance feature set fails to meet these criteria, the applicant software will not be accepted for compliance use.

#### Reporting Test Results

For each test case, the TDV energy use of the modeled building is reported (kBtu/ft2), along with the TDV energy use attributed to the major fuel types (electricity, gas), site energy use, and energy end use intensity for the regulated end uses (cooling, heating, lighting, etc.).  The following energy totals are reported:

- Annual TDV EUI (kBtu/ft2)

- Annual Site EUI – Electricity (kWh/ft2)

- Annual SiteEUI – Natural Gas (therm/ft2)

- Annual Total End Use Site Energy EUI – kBtu/ft2

Site Energy End Uses

- Site Energy: Heating (kBtu/ft2)

- Site Energy: Cooling (kBtu/ft2)

- Site Energy: Interior Lighting (kBtu/ft2)

- Site Energy: Interior Equipment (kBtu/ft2)

- Site Energy: Fans (kBtu/ft2) (Airside Fans, does not include tower fans)

- Site Energy: Pumps (kBtu/ft2)

- Site Energy: Towers (kBtu/ft2)Water heating (kBtu/ft2)

- TDV % Variation – this field is used for the compliance test

- Total End Use Site Energy % - percentage change in site energy use

- Pass/Fail – test fails if it does not meet passing criteria

- Unmet load hours – these are defined as the zone with the most unmet load hours

  - Reference Model Occupied Unmet Load Hours

  - Applicant Model Occupied Unmet Load Hours

Figure 1 – Results Spreadsheet Excerpt from Appendix 3B

The results spreadsheet provides the results of the Reference Method for each test, and provides a column (in orange) for the vendor to report the results from their candidate compliance software.

The variation from baseline section of the spreadsheet shows the percent change in TDV energy use (kBtu/ft2) from the base case for testing.  The percentage must be within the passing criteria for the candidate software to pass this test.

Also reported is the number of unmet load hours during occupied hours for the building. An unmet load hour for Title 24 compliance is defined as any hour when any of the zones served by the system has an unmet cooling or heating load.  This is typically reported by the software for each zone in the building.

The spreadsheet where the results are documented indicates whether the candidate software passes or fails a test. The result in column AH of the spreadsheet indicates whether or not the candidate software passes the test.

#### Software Sensitivity Test Cases

Test cases assess the energy impact of one or more of the building or system input characteristics on the baseline model. Each test suite consists of a series of unique test cases aimed to test the impact of a specific characteristic on building energy performance. Simulations are grouped according to test criteria and sub-grouped based on the reference model type to allow for direct comparison of results.  For each test case, the applicant software will modify the applicant baseline model with specific inputs as described in the Test Case description section.

The test cases are simulated on multiple California weather files to evaluate the sensitivity of the building or system input to extremes in climate. Results of the test case runs and the TDV percent variation over the baseline run shall be compiled and compared against the reference results.

A total of 106 test cases shall be performed. The baseline runs for each of the test cases are provided in Table 2 – Baseline Runs for Test Cases. Detailed descriptions of the Standard Design models are provided in Appendix 3A. EnergyPlus input files, Results output in html format and HVAC line diagram in svg format for all baseline and test case models are available from the California Energy Commission’s, Building Energy Efficiency Software Consortium webpage http://bees.archenergy.com

Table 2 – Baseline Runs for Test Cases

| Test Run Name | Baseline | Test Description |
| --- | --- | --- |
| 01CZ15StandAloneRetail-EnvelopeRoofInsulation | CZ15StandAloneRetail | Decrease overall U value of Roof by 20% compared to baseline case |
| 02CZ15StandAloneRetail Envelope WallInsulation | CZ15StandAloneRetail | Decrease overall U value of Exterior Wall by 20% compared to baseline case |
| 03CZ15StandAloneRetail Envelope Heavy | CZ15StandAloneRetail | Change Roof and Wall construction assembly to Wood Framed Roof and Heavy Mass wall. Roof and Wall assembly overall U value of test case= Baseline construction assembly U value |
| 04CZ16StandAloneRetail Envelope RoofInsulation | CZ16StandAloneRetail | Decrease overall U value of Roof by 20% compared to baseline case |
| 05CZ16StandAloneRetail Envelope WallInsulation | CZ16StandAloneRetail | Decrease overall U value of Exterior Wall by 20% compared to baseline case |
| 06CZ16StandAloneRetail Envelope Heavy | CZ16StandAloneRetail | Change Roof and Wall construction assembly to Wood Framed Roof and Heavy Mass wall. Roof and Wall assembly overall U value of test case= Baseline construction assembly U value |
| 07CZ06StandAloneRetail Envelope RoofInsulation | CZ06StandAloneRetail | Decrease overall U value of Roof by 20% compared to baseline case |
| 08CZ06StandAloneRetail Envelope WallInsulation | CZ06StandAloneRetail | Decrease overall U value of Exterior Wall by 20% compared to baseline case |
| 09CZ06StandAloneRetail Envelope Heavy | CZ06StandAloneRetail | Change Roof and Wall construction assembly to Wood Framed Roof and Heavy Mass wall. Roof and Wall assembly overall U value of test case= Baseline construction assembly U value |
| 10CZ15MediumOffice Envelope FloorslabInsulation | CZ15MediumOffice | Change Floor slab F factor to 0.45 |
| 11CZ15MediumOffice Envelope Infiltration | CZ15MediumOffice | Increase Exterior Wall Infiltration by 10% compared to baseline case |
| 12CZ15MediumOffice Glazing WindowU | CZ15MediumOfficeBaseline | Decrease U value of windows by 20% compared to baseline case |
| 13CZ15MediumOffice Glazing WindowSHGC | CZ15MediumOfficeBaseline | Decrease SHGC of windows by 20% compared to baseline case |
| 14CZ15MediumOffice Glazing WindowUSHGC | CZ15MediumOfficeBaseline | Decrease U value & SHGC of windows by 20% compared to baseline case |
| 15CZ16MediumOffice Envelope FloorslabInsulation | CZ16MediumOffice | Change Floor slab F factor to 0.45 |
| 16CZ16MediumOffice Envelope Infiltration | CZ16MediumOffice | Increase Exterior Wall Infiltration by 10% compared to baseline case |
| 17CZ16MediumOffice Glazing WindowU | CZ16MediumOfficeBaseline | Decrease U value of windows by 20% compared to baseline case |
| 18CZ16MediumOffice Glazing WindowSHGC | CZ16MediumOfficeBaseline | Decrease SHGC of windows by 20% compared to baseline case |
| 19CZ16MediumOffice Glazing WindowUSHGC | CZ16MediumOfficeBaseline | Decrease U value & SHGC of windows by 20% compared to baseline case |
| 20CZ06MediumOffice Envelope FloorslabInsulation | CZ06MediumOffice | Change Floor slab F factor to 0.45 |
| 21CZ06MediumOffice Envelope Infiltration | CZ06MediumOffice | Increase Exterior Wall Infiltration by 10% compared to baseline case |
| 22CZ06MediumOffice Glazing WindowU | CZ06MediumOffice | Decrease U value of windows by 20% compared to baseline case |
| 23CZ06MediumOffice Glazing WindowSHGC | CZ06MediumOffice | Decrease SHGC of windows by 20% compared to baseline case |
| 24CZ06MediumOffice Glazing WindowUSHGC | CZ06MediumOffice | Decrease U value & SHGC of windows by 20% compared to baseline case |
| 25CZ07LargeOffice WWR20% | CZ07LargeOffice | Reduce window area such that overall window to wall ratio is 20% |
| 26CZ07LargeOffice WWR60% | CZ07LargeOffice | Increase window area such that overall window to wall ratio is 60% |
| 27CZ07LargeOffice Glazing WWR60%SouthOverhang | CZ07LargeOffice | Increase window area such that overall window to wall ratio is 60%. Include horizontal overhang of two feet for all south facing windows |
| 28CZ01LargeOffice WWR20% | CZ01LargeOffice | Reduce window area such that overall window to wall ratio is 20% |
| 29CZ01LargeOffice WWR60% | CZ01LargeOffice | Increase window area such that overall window to wall ratio is 60% |
| 30CZ15MediumOffice Lighting LowLPD | CZ15MediumOffice | Decrease LPD by 20% compared to baseline case |
| 31CZ15MediumOffice Lighting HIghLPD | CZ15MediumOffice | Increase LPD by 20% compared to baseline case |
| 32CZ16MediumOffice Lighting LowLPD | CZ16MediumOffice | Decrease LPD by 20% compared to baseline case |
| 33CZ16MediumOffice Lighting HIghLPD | CZ16MediumOffice | Increase LPD by 20% compared to baseline case |
| 34CZ06MediumOffice Lighting LowLPD | CZ06MediumOffice | Decrease LPD by 20% compared to baseline case |
| 35CZ06MediumOffice Lighting HIghLPD | CZ06MediumOffice | Increase LPD by 20% compared to baseline case |
| 36CZ15MediumOffice Int.Eqpt LowEPD | CZ15MediumOffice | Decrease EPD by 20% compared to baseline case |
| 37CZ15MediumOffice Int.Eqpt HighEPD | CZ15MediumOffice | Increase EPD by 20% compared to baseline case |
| 38CZ16MediumOffice Int.Eqpt LowEPD | CZ16MediumOffice | Decrease EPD by 20% compared to baseline case |
| 39CZ16MediumOffice Int.Eqpt HighEPD | CZ16MediumOffice | Increase EPD by 20% compared to baseline case |
| 40CZ06MediumOffice Int.Eqpt LowEPD | CZ06MediumOffice | Decrease EPD by 20% compared to baseline case |
| 41CZ06MediumOffice Int.Eqpt HighEPD | CZ06MediumOffice | Increase EPD by 20% compared to baseline case |
| 42CZ07LargeOffice Daylighting Cont.DimHighVT | CZ07LargeOffice | Increase Visible Transmittance of windows by 20% compared to baseline case |
| 43CZ07LargeOffice Daylighting StepDim | CZ07LargeOffice | Change daylighting controls in Primary and Secondary daylit areas to Stepped dimming with 4 control steps |
| 44CZ07LargeOffice Daylighting StepDimHighVT | CZ07LargeOffice | Change daylighting controls in Primary and Secondary daylit areas to Stepped dimming with 4 control steps and increase visible transmittance of windows by 20% |
| 45CZ07LargeOffice Daylighting WWR20%Cont.DimHighVT | 25CZ07LargeOffice WWR20% | Increase Visible Transmittance of windows by 20% compared to baseline case. Reduce window area such that overall window to wall ratio is 20% Note for this test, Run 25CZ07LargeOfficeWWR20% is the baseline |
| 46CZ07LargeOffice Daylighting WWR20%StepDim | 25CZ07LargeOffice WWR20% | Change daylighting controls in Primary and Secondary daylit areas to Stepped dimming with 4 control steps.Reduce window area such that overall window to wall ratio is 20% Note for this test, Run 25CZ07LargeOfficeWWR20% is the baseline |
| 47CZ07LargeOffice Daylighting WWR20%StepDimHighVT | 25CZ07LargeOffice WWR20% | Change daylighting controls in Primary and Secondary daylit areas to Stepped dimming with 4 control steps and increase visible transmittance of windows by 20%. Reduce window area such that overall window to wall ratio is 0.2 Note for this test, Run 25CZ07LargeOfficeWWR0.2 is the baseline |
| 48CZ07LargeOffice Daylighting WWR60%Cont.DimHighVT | 26CZ07LargeOffice WWR60% | Increase Visible Transmittance of windows by 20% compared to baseline case. Reduce window area such that overall window to wall ratio is 60% Note for this test, Run 26CZ07LargeOfficeWWR60% is the baseline |
| 49CZ07LargeOffice Daylighting WWR60%StepDim | 26CZ07LargeOffice WWR60% | Change daylighting controls in Primary and Secondary daylit areas to Stepped dimming with 4 control steps.Reduce window area such that overall window to wall ratio is 60% Note for this test, Run 26CZ07LargeOffice WWR60% is the baseline |
| 50CZ07LargeOffice Daylighting WWR60%StepDimHighVT | 26CZ07LargeOffice WWR60% | Change daylighting controls in Primary and Secondary daylit areas to Stepped dimming with 4 control steps and increase visible transmittance of windows by 20%. Reduce window area such that overall window to wall ratio is 60% Note for this test, Run 26CZ07LargeOffice WWR60% is the baseline |
| 51CZ01LargeOffice Daylighting Cont.DimHighVT | CZ01LargeOffice | Increase Visible Transmittance of windows by 20% compared to baseline case |
| 52CZ01LargeOffice Daylighting StepDim | CZ01LargeOffice | Change daylighting controls in Primary and Secondary daylit areas to Stepped dimming with 4 control steps |
| 53CZ01LargeOffice Daylighting StepDimHighVT | CZ01LargeOffice | Change daylighting controls in Primary and Secondary daylit areas to Stepped dimming with 4 control steps and increase visible transmittance of windows by 20% |
| 54CZ01LargeOffice Daylighting WWR20%Cont.DimHighVT | 28CZ07LargeOffice WWR20% | Increase Visible Transmittance of windows by 20% compared to baseline case. Reduce window area such that overall window to wall ratio is 20% Note for this test, Run 28CZ07LargeOffice Glazing WWR20% is the baseline |
| 55CZ01LargeOffice Daylighting WWR20%StepDim | 28CZ07LargeOffice WWR20% | Change daylighting controls in Primary and Secondary daylit areas to Stepped dimming with 4 control steps. Reduce window area such that overall window to wall ratio is 20% Note for this test, Run 28CZ07LargeOfficeWWR20% is the baseline |
| 56CZ01LargeOffice Daylighting WWR20%StepDimHighVT | 28CZ07LargeOffice WWR20% | Change daylighting controls in Primary and Secondary daylit areas to Stepped dimming with 4 control steps and increase visible transmittance of windows by 20%. Reduce window area such that overall window to wall ratio is 20% Note for this test, Run 28CZ07LargeOffice WWR20% is the baseline |
| 57CZ01LargeOffice Daylighting WWR60%Cont.DimHighVT | 29CZ07LargeOffice WWR60% | Increase Visible Transmittance of windows by 20% compared to baseline case. Reduce window area such that overall window to wall ratio is 60% Note for this test, Run 29CZ07LargeOffice WWR60% is the baseline |
| 58CZ01LargeOffice Daylighting WWR60%StepDim | 29CZ07LargeOffice WWR60% | Change daylighting controls in Primary and Secondary daylit areas to Stepped dimming with 4 control steps. Reduce window area such that overall window to wall ratio is 60% Note for this test, Run 29CZ07LargeOffice WWR60% is the baseline |
| 59CZ01LargeOffice Daylighting WWR60%StepDimHighVT | 29CZ07LargeOffice WWR60% | Change daylighting controls in Primary and Secondary daylit areas to Stepped dimming with 4 control steps and increase visible transmittance of windows by 20%. Reduce window area such that overall window to wall ratio is 60% Note for this test, Run 29CZ07LargeOffice WWR60% is the baseline |
| 60CZ07StandAloneRetail Daylighting SRRBaseHighVT | CZ07StandAloneRetail | Increase Visible Transmittance of skylights by 20% compared to baseline case |
| 61CZ07StandAloneRetail Daylighting SRR3.04 | CZ07StandAloneRetail | Increase skylight area such that overall skylight to roof ratio is 3.04. |
| 62CZ07StandAloneRetail Daylighting SRR3.04HighVT | CZ07StandAloneRetail | Increase skylight area such that overall skylight to roof ratio is 3.04 and visible transmittance of skylights by 20% compared to baseline case. |
| 63CZ01StandAloneRetail Daylighting SRRBaseHighVT | CZ01StandAloneRetail | Increase Visible Transmittance of skylights by 20% compared to baseline case |
| 64CZ01StandAloneRetail Daylighting SRR3.04 | CZ01StandAloneRetail | Increase skylight area such that overall skylight to roof ratio is 3.04. |
| 65CZ01StandAloneRetail Daylighting SRR3.04HighVT | CZ01StandAloneRetail | Increase skylight area such that overall skylight to roof ratio is 3.04 and visible transmittance of skylights by 20% compared to baseline case |
| 66CZ16StandAloneRetail HVAC DuctLoss | CZ16StandAloneRetailDuctlossBaseline | Increase duct loss in the zones served by Packaged Single Zone systems to 14%. The baseline for this test is a modified version of Stand-Alone Retail prototype. See details below |
| 67CZ06StandAloneRetail HVAC DuctLoss | CZ06StandAloneRetailDuctlossBaseline | Increase duct loss in the zones served by Packaged Single Zone systems to 14%. The baseline for this test is a modified version of Stand-Alone Retail prototype. See details below |
| 68CZ15MediumOffice HVACPVAV Design | CZ15MediumOffice | Change static pressure and motor efficiency of all VAV fans. See details below |
| 69CZ15MediumOffice HVACPVAV SATControl | CZ15MediumOffice | Change supply air temperature reset based on Outside air. See details below |
| 70CZ15MediumOffice HVACPVAV VAVControl | CZ15MediumOffice | Change VAV damper control to dual maximum. |
| 71CZ15MediumOffice HVACPVAV EconomizerType | CZ15MediumOffice | Change Economizer Type to Fixed Dry bulb with 70 F high limit |
| 72CZ16MediumOffice HVACPVAV Design | CZ16MediumOffice | Change static pressure and motor efficiency of all VAV fans. See details below |
| 73CZ16MediumOffice HVACPVAV SATControl | CZ16MediumOffice | Change supply air temperature reset based on Outside air. See details below |
| 74CZ16MediumOffice HVACPVAV VAVControl | CZ16MediumOffice | Change VAV damper control to dual maximum. |
| 75CZ16MediumOffice HVACPVAV EconomizerType | CZ16MediumOffice | Change Economizer Type to Fixed Dry bulb with 70 F high limit |
| 76CZ06MediumOffice HVACPVAV Design | CZ06MediumOffice | Change static pressure and motor efficiency of all VAV fans. See details below |
| 77CZ06MediumOffice HVACPVAV SATControl | CZ06MediumOffice | Change supply air temperature reset based on Outside air. See details below |
| 78CZ06MediumOffice HVACPVAV VAVControl | CZ06MediumOffice | Change VAV damper control to dual maximum. |
| 79CZ06MediumOffice HVACPVAV EconomizerType | CZ06MediumOffice | Change Economizer Type to Fixed Dry bulb with 70 F high limit |
| 80CZ15LargeOffice HVACVAV ChillerCOP | CZ15LargeOffice | Increase Chiller COP by 11% compared to baseline case |
| 81CZ15LargeOffice HVACVAV CHWdeltaT | CZ15LargeOffice | Change the Chilled water loop design temperature difference to 12F. See below for details |
| 82CZ15LargeOffice HVACVAV CWSetpoint | CZ15LargeOffice | Change condenser water setpoint reset (50F to 95F) based on Outdoor air wet bulb temperature |
| 83CZ15LargeOffice HVACVAV TowerEff | CZ15LargeOffice |  |
| 84CZ16LargeOffice HVACVAV ChillerCOP | CZ16LargeOffice | Increase Chiller COP by 11% compared to baseline case |
| 85CZ16LargeOffice HVACVAV CHWdeltaT | CZ16LargeOffice | Change the Chilled water loop design temperature difference to 12F. See below for details |
| 86CZ16LargeOffice HVACVAV CWSetpoint | CZ16LargeOffice | Change condenser water setpoint reset (50F to 95F) based on Outdoor air wet bulb temperature |
| 87CZ16LargeOffice HVACVAV TowerEff | CZ16LargeOffice |  |
| 88CZ06LargeOffice HVACVAV ChillerCOP | CZ06LargeOffice | Increase Chiller COP by 11% compared to baseline case |
| 89CZ06LargeOffice HVACVAV CHWdeltaT | CZ06LargeOffice | Change the Chilled water loop design temperature difference to 12F. See below for details |
| 90CZ06LargeOffice HVACVAV CWSetpoint | CZ06LargeOffice | Change condenser water setpoint reset (50F to 95F) based on Outdoor air wet bulb temperature |
| 91CZ06LargeOffice HVACVAV TowerEff | CZ06LargeOffice |  |
| 92CZ15StripMall HVACPSZ DXCOP | CZ15StripMall PSZ | Increase COP of DX coil to 3.8 |
| 93CZ15StripMall HVACPSZ HeatEff | CZ15StripMall PSZ | Increase efficiency of the Heating coil to 90% |
| 94CZ15StripMall HVACPSZ EconomizerControl | CZ15StripMall PSZ | Change Economizer from Integrated as in baseline to Non-Integrated. See below for details |
| 95CZ16StripMall HVACPSZ DXCOP | CZ16StripMall PSZ | Increase COP of DX coil to 3.8 |
| 96CZ16StripMall HVACPSZ HeatEff | CZ16StripMall PSZ | Increase efficiency of the Heating coil to 90% |
| 97CZ16StripMall HVACPSZ EconomizerControl | CZ16StripMall PSZ | Change Economizer from Integrated as in baseline to Non-Integrated. See below for details |
| 98CZ06StripMall HVACPSZ DXCOP | CZ06StripMall PSZ | Increase COP of DX coil to 3.8 |
| 99CZ06StripMall HVACPSZ HeatEff | CZ06StripMall PSZ | Increase efficiency of the Heating coil to 90% |
| 100CZ06StripMall HVACPSZ EconomizerControl | CZ06StripMall PSZ | Change Economizer from Integrated as in baseline to Non-Integrated. See below for details |
| 101CZ15StripMall HVACPTAC DXCOP | CZ15StripMall PTAC | Increase COP of DX coil to 3.8 |
| 102CZ16StripMall HVACPTAC DXCOP | CZ16StripMall PTAC | Increase COP of DX coil to 3.8 |
| 103CZ06StripMall HVACPTAC DXCOP | CZ06StripMall PTAC | Increase COP of DX coil to 3.8 |
| 104CZ15StripMall HVACPTAC vsFanCoil | CZ15StripMall PTAC | Comparison of PTAC system with Fan Coil System. For details on Fan Coil system, refer StripMall FanCoil |
| 105CZ16StripMall HVACPTAC vsFanCoil | CZ16StripMall PTAC | Comparison of PTAC system with Fan Coil System. For details on Fan Coil system, refer StripMall FanCoil |
| 106CZ06StripMall HVACPTAC vsFanCoil | CZ06StripMall PTAC | Comparison of PTAC system with Fan Coil System. For details on Fan Coil system, refer StripMall FanCoil |

#### Results Documentation

The applicant shall perform simulations for all tests specified above. Detailed description of each test case is provided in Appendix 3A and report results in the forms provided in Appendix 3B. Some of the prototype models have some variants of the baseline model. These include 1) Stand Alone Duct Loss baseline- a variant of the Stand-Alone Retail model 2) StripMall-PTAC model- a variant of StripMall- PSZ model and 3) StripMall- Fan Coil model- a variant of StripMall PSZ model. For details please refer Appendix 3A.

A set of three test cases are presented here as examples: one for building envelope, one for lighting and daylighting, and one for HVAC. The development of the other required test cases follows the same process.

Example Test Case: #23 Envelope Glazing Test Medium OfficeCZ-6 Window SHGC

For this test case the SHGC of all vertical fenestration is decreased by 20%. The prototype used for this test case is a medium office building.

Before the test cases are run, the first step is to generate the prototype models for the four reference buildings, which are required for all of the tests.  The seven prototype models are defined in the PrototypeModel spreadsheet of Appendix 3A. (Note: while many of the prototype model inputs are based on Title 24 prescriptive requirements, the prototype models do not exactly conform to minimum Title 24 requirements, but rather, are intended to test the sensitivity of the candidate software simulation results to common variations in building inputs

Step 1. Generate prototype models. The first step is to generate the prototype building for the medium office building.  The detailed specification of the medium office building is listed in Appendix A.  A portion of the inputs are shown in the Figure below.  The prototypes are defined for the reference models on the Prototype Model tab of Appendix 3A.

Figure 2 – Prototype Model Definition from Appendix 3A

The prototype model definition in the spreadsheet contains links to other input definitions:

Row 19, 26,45: Links to layer-by-layer exterior construction assembly definitions in the ConstructionAssembly tab

Row 52: links to layer-by-layer interior construction assembly definitions in the ConstructionAssembly tab

Row 85, 105, 107, 109, 112: links to fan and equipment hourly schedules in the Schedules tab

Row 112, 116, 119: links to space use interior load schedules in the Schedules tab

Step 2. Define base case and variation for test run.

The base case is defined as the starting point for each test. In many tests, the base case will be one of the prototype models. However, in some cases, a variation of the prototype may serve as the base case for the test.

For this test, the base case is found by looking at the Test Criteria spreadsheet:

Figure 3 – Base Case Definition from Appendix 3A

For this test, the Baseline field in row 3 of the TestCritieria tab shows that the baseline is CZ06MediumOffice, the medium office prototype in climate zone 6.

This same TestCritieria tab shows the input(s) to be verified, which are highlighted in purple. For this test, the solar heat gain coefficient of all vertical fenestration is reduced by 20%, from 0.25 to 0.20.

Figure 4 – Input Parameter Variation for Medium Office from Appendix 3A

Step 3. Run the base case model and generate test results.

Once the base case model is developed, the simulation is run and the results are recorded onto the spreadsheet of test cases, Appendix 3B.

The candidate software shall report electricity use by end use, gas use by end use, TDV energy and unmet load hours. For purposes of compliance, unmet load hours are defined at the zone level, and the zone with the greatest number of unmet load hours must pass the criteria specified in the sizing procedure.

For the Reference Tests, the following parameters can be found in the output summary (html) file:

Figure 4 – System Sizing Data for Test Runs

Step 4. Run the test case model (with the reduced solar heat gain coefficient) and report the results.

The model is re-run and the energy results and outputs are reported. The percentage change in energy use is reported.

Step 5. Report the change in TDV energy use from the base case as a percentage change.

The reported percentage change in energy use from the candidate compliance software must fall within the passing criteria for the Reference Method.