### REFERENCE METHOD

### COMPLIANCE MARGIN TESTS

### For 2013 ACM

### (Draft Report)

## Table of Contents

I.	Introduction	4

II.	Overview	4

III.	Reference Model	4

IV.	Climate Zones	6

V.	Labeling Computer Runs	6

VI.	Test Criteria	6

VII.	Compliance Margin Test Cases	7

VIII.	Results Documentation	17

## Introduction

This document details the eligibility requirements for an applicant simulation program for use as compliance software under the 2013 ACM.  A series of quantitative tests called the Compliance Margin Tests shall be performed and results compared against predetermined Reference results to demonstrate that the applicant software is acceptable for use in code compliance. There are a total of 168 tests. All the test cases described in Section VII shall be performed and results summarized in the forms contained in Appendix A.

## Overview

The applicant software shall perform a suite of Compliance Margin Test Cases to demonstrate that their performance is acceptable for code compliance.  The applicant software test results shall be compared against a base case called the Reference tests. The Reference Test case is the corresponding match of a particular test case simulated already on EnergyPlus (v.7.0) engine. The Reference Model results, as determined by the Commission, are tabulated in Appendix A.

The test cases will assess the applicant software’s sensitivity to various inputs ranging from envelope thermal conductance to HVAC system performance. Each case tests the impact of the input component on building end use energy and annual TDV. The following seven input components will be tested through a series of _ tests:

Opaque envelope

Glazing

Lighting

Daylighting

Receptacle loads

HVAC System Parameters

## Reference Model

The Reference Method tests are performed on seven Prototype Models which are a subset of the DOE Prototype Building Models developed by PNNL for analysis of ASHRAE Standard 90.1. The Reference Models are EnergyPlus model input files of the DOE prototype models, modified to comply with the requirements of Title-24 2008. The Reference Models will be the reference baseline runs for the test cases. The applicant software shall replicate the seven building models using the same inputs as the Reference Models. The models so replicated will be the applicant baseline models for the test cases.

A summary of the Reference Models is provided in Appendix A. Detailed EnergyPlus input files of the Reference models are available from the California Energy Commission’s Building Energy Efficiency Software Consortium webpage http://www.energydataweb.com/consortium/PACdocsNR.aspx.

Reference Models used for the test cases are:

Small Office Building:

fig 1- Small Office

This is a single story building of area 5500 square feet with punched windows distributed evenly across all facades. There are five thermal zones in this building served by packaged air conditioning units. For further details refer Appendix A.

fig.2- Medium Office

Medium Office Building:

- This Reference model is a three storied rectangular building with an overall area of 53600 square feet. It has a window to wall ratio of 33% with fenestration distributed evenly across all four facades. The zones are served by DX cooling and Gas furnace heating with electric reheat. For further details refer Appendix A.

fig.3- Large Office

Large Office Building:

- The large office building has twelve floors and a basement floor with glass windows with a window to wall ratio of 40% on the above-grade walls. The total area of the building is 498,600 square feet.  The HVAC system type used is centrifugal chillers and gas boiler. For further details refer Appendix A.

Stand-Alone Retail:

fig.4- Stand-Alone Retail

The stand-alone retail is a single story rectangular building measuring 178 ft by 139 ft. The total area is 24695 square feet. Windows are located only on the street facing façade and occupy 25.4% of that facade. The building is divided into five thermal zones which are served by various system types as described in Appendix A.

fig.5- Primary School

Primary School:

- The primary school building is a single story building with a total floor area of 73, 960 square feet. It has a window to wall ratio of 35% with ribbon windows across all facades. Thermal zoning is as per exposure and space function. The zones are served by various system types. For further details refer Appendix A.

Fast-food Restaurant:

fig.6- Fast food Restaurant

The fast food restaurant Model has one floor of area 2500 square feet. Windows are located on the east, south and west sides of the dining zone façade. Window to wall ratio varies for each façade. The building is served by packaged air conditioning unit. For further details refer Appendix A.

Warehouse Building:

- The warehouse building has a total area of 49,495 square feet. It has three zones with office and storage spaces. The office spaces have punched windows. For further details refer Appendix A.

fig.7- Warehouse

Strip Mall Building:

fig 8: Strip Mall Building

The stripmall building has a total area of 22,500 square feet. It has ten zones each with rooftop units. The building has windows in the street facing façade and has an overall window to wall ratio of 10.5% . For further details refer Appendix A.

## Climate Zones

The Reference Method Test cases use five of the 16 California climate zones. Most tests are performed on two or three climate zones to capture the sensitivity of the input characteristic to extremes in weather conditions. The test cases are performed on climate zones 6, 15 and 16 which represent mild, hot and cold climates respectively. Daylighting tests are performed on climate zone 7 which has a high annual daylighting potential and climate zone 1, representative of climate with least annual daylighting potential.

Table 1-Climate Zones Tested

| Climate Zone | Example City |
| --- | --- |
| 1 | Arcata |
| 6 | Long Beach |
| 7 | San Diego Lindbergh |
| 15 | Palm Springs-Intl |
| 16 | Blue Canyon |

## Labeling Computer Runs

Each test case used for the compliance test is labeled uniquely to make it easier to keep track of the runs and to facilitate analysis. The following scheme is used:

Envelope Test/Medium Office -CZ-15, Roof Insulation

Input Characteristic

Climate Zone

Prototype Model Type

Test Component

## Test Criteria

Applicant software vendors shall perform a series of computer runs. Each of these runs shall be a systematic variation of the applicant base case model as described in Section VII. The applicant test case results will be compared to the reference results to verify that applicant software meets the requirements of the ACM. Simulation results for each test case will be compiled in forms provided in Appendix A. Compiled results will include annual site energy consumption for each end-use, overall site energy consumption, total unmet load hours and annual TDV and % variation of annual TDV and total end use site energy.

The annual TDV % variation shall be calculated using the formula:

TDV%= (TDVb – TDVn)/TDVb

Where, TDV% is the TDV % variation,

TDVn is the annual TDV for test case number n and

TDVb is the annual TDV for the base case run.

In order to be accepted, the applicant software should fulfill the passing criteria as determined by the Commission:

For each test case,

- When Reference Model TDV%r  ≤ 1%, Allowed Maximum Applicant Model TDV%a  =1%

- When Reference Model TDV% r > 1%, Allowed Applicant Model TDV%a  shall be within ± 10% of Reference Model TDV%r

Where,   the subscript “a” represents the applicant results and “r” represents the reference results.

If any of the tests fail to meet these criteria, the applicant software will not be accepted for compliance use.

## Compliance Margin Test Cases

Test cases assess the energy impact of one or more of the building or system input characteristics on the baseline model. Each test suite consists of a series of unique test cases aimed to test the impact of a specific characteristic on building energy performance. Simulations are grouped according to test criteria and sub-grouped based on the reference model type to allow for direct comparison of results.  For each test case, the applicant software will modify the applicant baseline model with specific inputs as described in the Test Case description section.

The test cases are simulated on multiple California weather files to evaluate the sensitivity of the building or system input to extremes in climate. Results of the test case runs and the TDV percent variation over the baseline run shall be compiled and compared against the reference results.

Each test case described in this section shall be compared against a baseline run. The baseline runs for each of the test cases are provided in Table 2- Baseline Runs for Test Cases. A summary of the baseline reference Models is provided in Appendix A. Detailed EnergyPlus input files of the Reference baseline and test case models are available from the California Energy Commission’s, Building Energy Efficiency Software Consortium webpage http://www.energydataweb.com/consortium/PACdocsNR.aspx.

Test cases:

Envelope Test

- Envelope Test/Medium Office-CZ-15, Roof Insulation- This case tests the impact of modifying Roof assembly overall U value in climate zone 15. Roof assembly U value is increased 20% more than the baseline case.

- Envelope Test/Medium Office-CZ-15, Wall Insulation- This case tests the impact of modifying Wall assembly overall U value in climate zone 15. Wall assembly U value is modeled 20% more than the baseline case.

- Envelope Test/Medium Office-CZ-15, Floor Slab Insulation- This case tests the impact of a slab on grade floor with vertical perimeter insulation in climate zone 15. The baseline case has floor slab with no insulation.

- Envelope Test/Medium Office-CZ-15, Infiltration- This case tests the impact of increasing the envelope infiltration by 5% compared to baseline case in climate zone 15.

- Envelope Test/Medium Office-CZ-16, Roof Insulation- This case tests the impact of modifying Roof assembly overall U value in climate zone 16. Roof assembly U value is increased 20% more than the baseline case.

- Envelope Test/Medium Office-CZ-16, Wall Insulation- This case tests the impact of modifying Wall assembly overall U value in climate zone 15. Wall assembly U value is modeled 20% more than the baseline case.

- Envelope Test/Medium Office-CZ-16, Floor Slab Insulation- This case tests the impact of a slab on grade floor with vertical perimeter insulation in climate zone 15. The baseline case has floor slab with no insulation.

- Envelope Test/Medium Office-CZ-16, Infiltration- This case tests the impact of increasing the envelope infiltration by 5% compared to baseline case in climate zone 16.

- Envelope Test/Medium Office-CZ-6, Roof Insulation- This case tests the impact of modifying Roof assembly overall U value in climate zone 6. Roof assembly U value is increased 20% more than the baseline case.

- Envelope Test/Medium Office-CZ-6, Wall Insulation- This case tests the impact of modifying Wall assembly overall U value in climate zone 6. Wall assembly U value is modeled 20% more than the baseline case.

- Envelope Test/Medium Office-CZ-6, Floor Slab Insulation- This case tests the impact of a slab on grade floor with vertical perimeter insulation in climate zone 6. The baseline case has floor slab with no insulation.

- Envelope Test/Medium Office-CZ-6, Infiltration- This case tests the impact of increasing the envelope infiltration by 5% compared to baseline case in climate zone 6.

- Envelope Test/Small Office-CZ-15, Low Sloped Roof, Solar Reflectance- This case tests the impact of modifying the reflectivity of the roof in climate zone -15. Here, aged solar reflectance of the roofing product is increased by 20% compared to a low sloped base case roof.

- Envelope Test/Small Office-CZ-16, Low Sloped Roof, Solar Reflectance- This case tests the impact of modifying the reflectivity of the roof in climate zone -16. Here, aged solar reflectance of the roofing product is increased by 20% compared to a low sloped base case roof.

- Envelope Test/Small Office-CZ-6, Low Sloped Roof, Solar Reflectance- This case tests the impact of modifying the reflectivity of the roof in climate zone -6. Here, aged solar reflectance of the roofing product is increased by 20% compared to a low sloped base case roof.

- Envelope Test/Small Office-CZ-15, Steep Sloped Roof, Solar Reflectance- This case tests the impact of modifying the reflectivity of the roof in climate zone -15. Here, aged solar reflectance of the roofing product is increased by 20% compared to a steep sloped base case roof.

- Envelope Test/Small Office-CZ-16, Steep Sloped Roof, Solar Reflectance- This case tests the impact of modifying the reflectivity of the roof in climate zone -16. Here, aged solar reflectance of the roofing product is increased by 20% compared to a steep sloped base case roof.

- Envelope Test/Small Office-CZ-6, Steep Sloped Roof, Solar Reflectance- This case tests the impact of modifying the reflectivity of the roof in climate zone -6. Here, aged solar reflectance of the roofing product is increased by 20% compared to a low sloped base case roof.

- Envelope Test/Primary School-CZ-15-Heavy Construction- In this case, the impact of envelope mass on energy consumption in climate zone 15 is tested. The test case is modeled with a Solid concrete wall and compared against the light weight construction baseline.

- Envelope Test/Primary School-CZ-16-Heavy Construction- In this case, the impact of envelope mass on energy consumption in climate zone 16 is tested. The test case is modeled with a Solid concrete wall and compared against the light weight construction baseline.

- Envelope Test/Primary School-CZ-6-Heavy Construction- In this case, the impact of envelope mass on energy consumption in climate zone 6 is tested. The test case is modeled with a Solid concrete wall and compared against the light weight construction baseline.

Envelope Glazing Test

- Envelope Glazing Test/Medium Office-CZ-15, Window U value- This case tests the impact of reducing the window U value by 20% compared to baseline window U value on climate zone 15. The intent is to test the sensitivity of the software to modifications in heat transmitted into the space through fenestration.

- Envelope Glazing Test/Medium Office-CZ-15, Window SHGC value- This case tests the impact of reducing the window SHGC value by 20% compared to baseline SHGC on climate zone 15. The intent is to test the sensitivity of the software to modifications in heat transmitted into the space through fenestration.

- Envelope Glazing Test/Medium Office-CZ-15, Window U & SHGC value- This case tests the impact of reducing the window U  and SHGC values by 20% compared to baseline on climate zone 15. The intent is to test the sensitivity of the software to modifications in heat transmitted into the space through fenestration.

- Envelope Glazing Test/Medium Office-CZ-16, Window U value- This case tests the impact of reducing the window U value by 20% compared to baseline window U value on climate zone 16. The intent is to test the sensitivity of the software to modifications in heat transmitted into the space through fenestration.

- Envelope Glazing Test/Medium Office-CZ-16, Window SHGC value- This case tests the impact of reducing the window SHGC value by 20% compared to baseline SHGC on climate zone 16. The intent is to test the sensitivity of the software to modifications in heat transmitted into the space through fenestration.

- Envelope Glazing Test/Medium Office-CZ-16, Window U & SHGC value- This case tests the impact of reducing the window U and SHGC values by 20% compared to baseline on climate zone 16. The intent is to test the sensitivity of the software to modifications in heat transmitted into the space through fenestration.

- Envelope Glazing Test/Medium Office-CZ-6, Window U value- This case tests the impact of reducing the window U value by 20% compared to baseline window U value on climate zone 6. The intent is to test the sensitivity of the software to modifications in heat transmitted into the space through fenestration.

- Envelope Glazing Test/Medium Office-CZ-6, Window SHGC value- This case tests the impact of reducing the window U value by 20% compared to baseline window U value on climate zone 6. The intent is to test the sensitivity of the software to modifications in heat transmitted into the space through fenestration.

- Envelope Glazing Test/Medium Office-CZ-6, Window U & SHGC value- This case tests the impact of reducing the window U and SHGC values by 20% compared to baseline on climate zone 6. The intent is to test the sensitivity of the software to modifications in heat transmitted into the space through fenestration.

- Envelope Glazing Test/Large Office-CZ-7, WWR of 20%- In this case, the impact of reducing the window to wall ratio is tested. The test case is compared against the baseline which has a WWR of 40%. This case is tested for climate zone 7.

- Envelope Glazing Test/Large Office-CZ-7, WWR of 60%- In this case, the impact of increasing the window to wall ratio is tested. The test case is compared against the baseline which has a WWR of 40%. This case is tested for climate zone 7.

- Envelope Glazing Test/Large Office-CZ-7, WWR of 60% with overhang on South Façade- In this case, the impact of shading the south facing windows by a two feet deep overhang is tested. This case is tested for climate zone 7.

- Envelope Glazing Test/Large Office-CZ-1, WWR of 20%- In this case, the impact of reducing the window to wall ratio is tested. The test case is compared against the baseline which has a WWR of 40%. This case is tested for climate zone 1.

- Envelope Glazing Test/Large Office-CZ-1, WWR of 60%- In this case, the impact of increasing the window to wall ratio is tested. The test case is compared against the baseline which has a WWR of 40%. This case is tested for climate zone 1.

Lighting Test

- Lighting Test/Medium Office-CZ-15, low lighting power density- This case tests the impact of reducing the lighting power density of interior lights by 20% compared to baseline. This case is tested on climate zone 15.

- Lighting Test/Medium Office-CZ-15, high lighting power density- This case tests the impact of increasing the lighting power density of interior lights by 20% compared to baseline. This case is tested on climate zone 15.

- Lighting Test/Medium Office-CZ-16, low lighting power density- This case tests the impact of reducing the lighting power density of interior lights by 20% compared to baseline. This case is tested on climate zone 16.

- Lighting Test/Medium Office-CZ-16, high lighting power density- This case tests the impact of increasing the lighting power density of interior lights by 20% compared to baseline. This case is tested on climate zone 16.

- Lighting Test/Medium Office-CZ-6, low lighting power density- This case tests the impact of reducing the lighting power density of interior lights by 20% compared to baseline. This case is tested on climate zone 6.

- Lighting Test/Medium Office-CZ-6, high lighting power density- This case tests the impact of increasing the lighting power density of interior lights by 20% compared to baseline. This case is tested on climate zone 6.

Daylighting Test

- Daylighting Test/Large Office-CZ-7, Daylight sensors with Continuous Dimming- This case tests the impact of daylighting. Daylight sensors with continuous dimming is modeled in a large office building type with a WWR of 40% and results compared against Large office Prototype model , both modeled  for climate zone 7.

- Daylighting Test/Large Office-CZ-7, Daylight sensors with Continuous Dimming and windows with high VT- This case tests the impact of daylighting. Daylight sensors with continuous dimming are modeled in a large office building type with a WWR of 40%. The visible transmittance of windows is increased to improve the daylighting potential. This case is tested on large office Prototype model, both modeled for climate zone 7.

- Daylighting Test/Large Office-CZ-7, Daylight sensors with Stepped Dimming- This case tests the impact of daylighting. Daylight sensors with stepped dimming is modeled in a large office building type with a WWR of 40% and results compared against Large office Prototype model, both modeled  for climate zone 7.

- Daylighting Test/Large Office-CZ-7, Daylight sensors with Stepped Dimming and windows with high VT- This case tests the impact of daylighting. Daylight sensors with stepped dimming are modeled in a large office building type with a WWR of 40%. The visible transmittance of windows is increased to improve the daylighting potential. This case is tested on large office Prototype model, both modeled for climate zone 7.

- Daylighting Test/Large Office-CZ-7,WWR of 20%, Daylight sensors with Continuous Dimming- This case tests the impact of daylighting. Daylight sensors with continuous dimming is modeled in a large office building type with a WWR of 20% and results compared against Large office Prototype model , both modeled  for climate zone 7.

- Daylighting Test/Large Office-CZ-7, WWR of 20%, Daylight sensors with Continuous Dimming and windows with high VT--This case tests the impact of daylighting. Daylight sensors with continuous dimming are modeled in a large office building type with a WWR of 20%. The visible transmittance of windows is increased to improve the daylighting potential. This case is tested on large office Prototype model in climate zone 7.

- Daylighting Test/Large Office-CZ-7, WWR of 20%, Daylight sensors with Stepped Dimming- This case tests the impact of daylighting. Daylight sensors with stepped dimming is modeled in a large office building type with a WWR of 20% and results compared against Large office Prototype model  in climate zone 7.

- Daylighting Test/Large Office-CZ-7, WWR of 20%, Daylight sensors with Stepped Dimming and windows with high VT- This case tests the impact of daylighting. Daylight sensors with stepped dimming are modeled in a large office building type with a WWR of 20%. The visible transmittance of windows is increased to improve the daylighting potential. This case is tested on large office Prototype model in climate zone 7.

- Daylighting Test/Large Office-CZ-7,WWR of 60%, Daylight sensors with Continuous Dimming- This case tests the impact of daylighting. Daylight sensors with continuous dimming is modeled in a large office building type with a WWR of 60% and results compared against Large office Prototype model  in climate zone 7.

- Daylighting Test/Large Office-CZ-7, WWR of 60%, Daylight sensors with Continuous Dimming and windows with high VT- This case tests the impact of daylighting. Daylight sensors with continuous dimming are modeled in a large office building type with a WWR of 60%. The visible transmittance of windows is increased to improve the daylighting potential. This case is tested on large office Prototype model in climate zone 7.

- Daylighting Test/Large Office-CZ-7, WWR of 60%, Daylight sensors with Stepped Dimming- This case tests the impact of daylighting. Daylight sensors with stepped dimming is modeled in a large office building type with a WWR of 60% and results compared against Large office Prototype model  in climate zone 7.

- Daylighting Test/Large Office-CZ-7, WWR of 60%, Daylight sensors with Stepped Dimming and windows with high VT- This case tests the impact of daylighting. Daylight sensors with stepped dimming are modeled in a large office building type with a WWR of 60%. The visible transmittance of windows is increased to improve the daylighting potential. This case is tested on large office Prototype model in climate zone 7.

- Daylighting Test/Large Office-CZ-1, Daylight sensors with Continuous Dimming- This case tests the impact of daylighting. Daylight sensors with continuous dimming is modeled in a large office building type with a WWR of 40% and results compared against Large office Prototype model , both modeled  for climate zone 1.

- Daylighting Test/Large Office-CZ-1, Daylight sensors with Continuous Dimming and windows with high VT- This case tests the impact of daylighting. Daylight sensors with continuous dimming are modeled in a large office building type with a WWR of 40%. The visible transmittance of windows is increased to improve the daylighting potential. This case is tested on large office Prototype model, both modeled for climate zone 1.

- Daylighting Test/Large Office-CZ-1, Daylight sensors with Stepped Dimming- This case tests the impact of daylighting. Daylight sensors with stepped dimming is modeled in a large office building type with a WWR of 40% and results compared against Large office Prototype model , both modeled  for climate zone 1.

- Daylighting Test/Large Office-CZ-1, Daylight sensors with Stepped Dimming and windows with high VT- This case tests the impact of daylighting. Daylight sensors with stepped dimming are modeled in a large office building type with a WWR of 40%. The visible transmittance of windows is increased to improve the daylighting potential. This case is tested on large office Prototype model, both modeled  for climate zone 1.

- Daylighting Test/Large Office-CZ-1,WWR of 20%, Daylight sensors with Continuous Dimming- This case tests the impact of daylighting. Daylight sensors with continuous dimming is modeled in a large office building type with a WWR of 20% and results compared against Large office Prototype model , both modeled  for climate zone 1.

- Daylighting Test/Large Office-CZ-1, WWR of 20%, Daylight sensors with Continuous Dimming and windows with high VT--This case tests the impact of daylighting. Daylight sensors with continuous dimming are modeled in a large office building type with a WWR of 20%. The visible transmittance of windows is increased to improve the daylighting potential. This case is tested on large office Prototype model in climate zone 1.

- Daylighting Test/Large Office-CZ-1, WWR of 20%, Daylight sensors with Stepped Dimming- This case tests the impact of daylighting. Daylight sensors with stepped dimming is modeled in a large office building type with a WWR of 20% and results compared against Large office Prototype model  in climate zone 1.

- Daylighting Test/Large Office-CZ-1, WWR of 20%, Daylight sensors with Stepped Dimming and windows with high VT- This case tests the impact of daylighting. Daylight sensors with stepped dimming are modeled in a large office building type with a WWR of 20%. The visible transmittance of windows is increased to improve the daylighting potential. This case is tested on large office Prototype model in climate zone 1.

- Daylighting Test/Large Office-CZ-1 ,WWR of 60%, Daylight sensors with Continuous Dimming- This case tests the impact of daylighting. Daylight sensors with continuous dimming is modeled in a large office building type with a WWR of 60% and results compared against Large office Prototype model  in climate zone 1.

- Daylighting Test/Large Office-CZ-1, WWR of 60%, Daylight sensors with Continuous Dimming and windows with high VT- This case tests the impact of daylighting. Daylight sensors with continuous dimming are modeled in a large office building type with a WWR of 60%. The visible transmittance of windows is increased to improve the daylighting potential. This case is tested on large office Prototype model in climate zone 1.

- Daylighting Test/Large Office-CZ-1, WWR of 60%, Daylight sensors with Stepped Dimming- This case tests the impact of daylighting. Daylight sensors with stepped dimming is modeled in a large office building type with a WWR of 60% and results compared against Large office Prototype model  in climate zone 1.

- Daylighting Test/Large Office-CZ-1, WWR of 60%, Daylight sensors with Stepped Dimming and windows with high VT- This case tests the impact of daylighting. Daylight sensors with stepped dimming are modeled in a large office building type with a WWR of 60%. The visible transmittance of windows is increased to improve the daylighting potential. This case is tested on large office Prototype model in climate zone 1.

- Daylighting Test/Stand Alone Retail-CZ-7, with skylights with high VT- In this case the impact of daylighting with skylights is tested. Daylight potential is improved by increasing the visible transmittance of the skylight glass. Standalone retail building in climate zone 7 is tested in this case.

- Daylighting Test/Stand Alone Retail-CZ-7, with SRR of 5%- In this case the impact of daylighting with skylights is tested. Daylight potential is improved by increasing the SRR to 5% compared to baseline SRR of 1.3%. Standalone retail building in climate zone 7 is tested in this case.

- Daylighting Test/Stand Alone Retail-CZ-7, with SRR of 5% and skylights with high VT - In this case the impact of daylighting with skylights is tested. Daylight potential is improved by increasing the SRR to 5% compared to baseline SRR of 1.3%, and visible transmittance of skylight glass. Standalone retail building in climate zone 7 is tested in this case.

- Daylighting Test/Stand Alone Retail-CZ-1, with skylights with high VT-- In this case the impact of daylighting with skylights is tested. Daylight potential is improved by increasing the visible transmittance of the skylight glass. Standalone retail building in climate zone 1 is tested in this case.

- Daylighting Test/Stand Alone Retail-CZ-1, with SRR of 5%-%- In this case the impact of daylighting with skylights is tested. Daylight potential is improved by increasing the SRR to 5% compared to baseline SRR of 1.3%. Standalone retail building in climate zone 1 is tested in this case.

- Daylighting Test/Stand Alone Retail-CZ-1, with SRR of 5% and skylights with high VT - In this case the impact of daylighting with skylights is tested. Daylight potential is improved by increasing the SRR to 5% compared to baseline SRR of 1.3%, and visible transmittance of skylight glass. Standalone retail building in climate zone 1 is tested in this case.

Equipment Test

- Equipment Test/Medium Office-CZ-15, with low Equipment Power Density- This case tests the impact of reducing the equipment power density by 20% compared to baseline. This case is tested on climate zone 15.

- Equipment Test/Medium Office-CZ-15, with high Equipment Power Density- This case tests the impact of increasing the equipment power density by 20% compared to baseline. This case is tested on climate zone 15.

- Equipment Test/Medium Office-CZ-16, with low Equipment Power Density- This case tests the impact of reducing the equipment power density by 20% compared to baseline. This case is tested on climate zone 16.

- Equipment Test/Medium Office-CZ-16, with high Equipment Power Density- This case tests the impact of increasing the equipment power density by 20% compared to baseline. This case is tested on climate zone 16.

- Equipment Test/Medium Office-CZ-6, with low Equipment Power Density- This case tests the impact of reducing the equipment power density by 20% compared to baseline. This case is tested on climate zone 6.

- Equipment Test/Medium Office-CZ-6, with high Equipment Power Density- This case tests the impact of increasing the equipment power density by 20% compared to baseline. This case is tested on climate zone 6.

- Equipment Test/Fast food Restaurant-CZ-6, with high Process Load- This case tests the impact of increasing the electric process load by 5% compared to baseline. This case is tested on climate zone 6.

- The System test suit comprises of test cases for 12 different HVAC system types like:

  - Packaged Terminal Air Conditioner

  - Packaged Terminal Heat Pump

  - Packaged Single Zone System

  - Packaged VAV System

  - Single duct built-up VAV system

  - Fan coil units

  - Water loop heat pump

  - Evaporative cooling system

  - Natural ventilation

  - Hydronic radiant system

  - Under-floor air distribution system

  - Variable refrigerant flow (VRF) system

- HVAC Test Cases

- HVAC Test /PTAC, Stripmall-CZ-6, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test /PTAC, Stripmall-CZ-6, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test /PTAC, Stripmall-CZ-15, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test /PTAC, Stripmall-CZ-15, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test /PTAC, Stripmall-CZ-16, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test /PTAC, Stripmall-CZ-16, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test /PTHP, Stripmall-CZ-6, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test /PTHP, Stripmall-CZ-6, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test /PTHP, Stripmall-CZ-15, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test /PTHP, Stripmall-CZ-15, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test /PTHP, Stripmall-CZ-16, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test /PTHP, Stripmall-CZ-16, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test/ PSZ, Stripmall-CZ-6, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test/ PSZ, Stripmall-CZ-6, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test/ PSZ, Stripmall-CZ-15, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test/ PSZ, Stripmall-CZ-15, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test/ PSZ, Stripmall-CZ-16, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test/ PSZ, Stripmall-CZ-16, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test/PVAV, Medium Office-CZ-6, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test/PVAV, Medium Office-CZ-6, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test/PVAV, Medium Office-CZ-15, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test/PVAV, Medium Office-CZ-15, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test/PVAV, Medium Office-CZ-16, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test/PVAV, Medium Office-CZ-16, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test /VAV, Large Office-CZ-6, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test /VAV, Large Office-CZ-6, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test /VAV, Large Office-CZ-15, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test /VAV, Large Office-CZ-15, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test /VAV, Large Office-CZ-16, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test /VAV, Large Office-CZ-16, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test/FanCoil, Stripmall-CZ-6, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test/FanCoil, Stripmall-CZ-6, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test/FanCoil, Stripmall-CZ-15, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test/FanCoil, Stripmall-CZ-15, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test/FanCoil, Stripmall-CZ-16, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test/FanCoil, Stripmall-CZ-16, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test/VRF , Stripmall-CZ-6, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test/VRF, Stripmall-CZ-6, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test/VRF , Stripmall-CZ-15, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test/VRF, Stripmall-CZ-15, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

- HVAC Test/VRF , Stripmall-CZ-16, Low Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements lower than the 2008 Title 24 baseline.

- HVAC Test/VRF, Stripmall-CZ-16, High Efficiency- This case tests the impact of modifying a set of parameters with efficiencies and requirements higher than the 2008 Title 24 baseline.

Table 2- Baseline Runs for Test Cases

| Test Case No. | Test Case | Baseline Model |
| --- | --- | --- |
| 1 | Envelope Test Medium OfficeCZ-15 Roof Insulation | Medium Office CZ-15 Baseline |
| 2 | Envelope Test Medium OfficeCZ-15 Wall Insulation |  |
| 3 | Envelope Test Medium OfficeCZ-15 Floor Slab Insulation |  |
| 4 | Envelope Test Medium OfficeCZ-15 Infiltration |  |
| 5 | Envelope Test Medium OfficeCZ-16 Roof Insulation | Medium Office CZ-16 Baseline |
| 6 | Envelope Test Medium OfficeCZ-16 Wall Insulation |  |
| 7 | Envelope Test Medium OfficeCZ-16 Floor Slab Insulation |  |
| 8 | Envelope Test Medium OfficeCZ-16 Infiltration |  |
| 9 | Envelope Test Medium OfficeCZ-6 Roof Insulation | Medium Office CZ-6 Baseline |
| 10 | Envelope Test Medium OfficeCZ-6 Wall Insulation |  |
| 11 | Envelope Test Medium OfficeCZ-6 Floor Slab Insulation |  |
| 12 | Envelope Test Medium OfficeCZ-6 Infiltration |  |
| 13 | Envelope Test Small OfficeCZ-15 SolarReflectance-Low Slope | Small Office CZ-15 SolarReflectance-Low Slope Baseline |
| 14 | Envelope Test Small OfficeCZ-16 SolarReflectance-Low Slope | Small Office CZ-16 SolarReflectance-Low Slope Baseline |
| 15 | Envelope Test Small OfficeCZ-6 SolarReflectance-Low Slope | Small Office CZ-6 SolarReflectance-Low Slope Baseline |
| 16 | Envelope Test Small OfficeCZ-15 SolarReflectance-Steep Slope | Small Office CZ-15 SolarReflectance-Steep Slope Baseline |
| 17 | Envelope Test Small OfficeCZ-16 SolarReflectance-Steep Slope | Small Office CZ-16 SolarReflectance-Steep Slope Baseline |
| 18 | Envelope Test Small OfficeCZ-6 SolarReflectance-Steep Slope | Small Office CZ-6 SolarReflectance-Steep Slope Baseline |
| 19 | Envelope Test Primary SchoolCZ-15 Heavy | Envelope Test Primary SchoolCZ-15 Light |
| 20 | Envelope Test Primary SchoolCZ-16 Heavy | Envelope Test Primary SchoolCZ-16 Light |
| 21 | Envelope Test Primary SchoolCZ-6 Heavy | Envelope Test Primary SchoolCZ-6 Light |
| 22 | Envelope Glazing Test Medium OfficeCZ-15 Window U | Medium Office CZ-15 Baseline |
| 23 | Envelope Glazing Test Medium OfficeCZ-15 Window SHGC |  |
| 24 | Envelope Glazing Test Medium OfficeCZ-15 Window U&SHGC |  |
| 25 | Envelope Glazing Test Medium OfficeCZ-16 Window U | Medium Office CZ-16 Baseline |
| 26 | Envelope Glazing Test Medium OfficeCZ-16 Window SHGC |  |
| 27 | Envelope Glazing Test Medium OfficeCZ-16 Window U&SHGC |  |
| 28 | Envelope Glazing Test Medium OfficeCZ-6 Window U | Medium Office CZ-6 Baseline |
| 29 | Envelope Glazing Test Medium OfficeCZ-6 Window SHGC |  |
| 30 | Envelope Glazing Test Medium OfficeCZ-6 Window U&SHGC |  |
| 31 | Envelope Glazing Test Large OfficeCZ-7 WWR 20% | Large Office CZ-7 Baseline |
| 32 | Envelope Glazing Test Large OfficeCZ-7 WWR 60% |  |
| 33 | Envelope Glazing Test Large OfficeCZ-7 WWR 60% with South overhang |  |
| 34 | Envelope Glazing Test Large OfficeCZ-1 WWR 20% |  |
| 35 | Envelope Glazing Test Large OfficeCZ-1 WWR 60% |  |
| 36 | Lighting Tests Medium OfficeCZ-15 low LPD | Medium Office CZ-15 Baseline |
| 37 | Lighting Tests Medium OfficeCZ-15 high LPD |  |
| 38 | Lighting Tests Medium OfficeCZ-16 low LPD | Medium Office CZ-16 Baseline |
| 39 | Lighting Tests Medium OfficeCZ-16 high LPD |  |
| 40 | Lighting Tests Medium OfficeCZ-6 low LPD | Medium Office CZ-6 Baseline |
| 41 | Lighting Tests Medium OfficeCZ-6 high LPD |  |
| 42 | Daylighting Tests Large OfficeCZ-7 Continuous Dimming | Large Office CZ-7 Baseline |
| 43 | Daylighting Tests Large OfficeCZ-7 Continuous Dimming and high VT |  |
| 44 | Daylighting Tests Large OfficeCZ-7 Stepped Dimming |  |
| 45 | Daylighting Tests Large OfficeCZ-7 Stepped Dimming and high VT |  |
| 46 | Daylighting Tests Large OfficeCZ-7 WWR-20%,Continuous Dimming | Large Office CZ-7 WWR 20% Baseline |
| 47 | Daylighting Tests Large OfficeCZ-7 WWR-20%,Continuous Dimming and high VT |  |
| 48 | Daylighting Tests Large OfficeCZ-7 WWR-20%,Stepped Dimming |  |
| 49 | Daylighting Tests Large OfficeCZ-7 WWR-20%,Stepped Dimming and high VT |  |
| 50 | Daylighting Tests Large OfficeCZ-7WWR-60%,Continuous Dimming | Large Office CZ-7 WWR 60% Baseline |
| 51 | Daylighting Tests Large OfficeCZ-7 WWR-60%,Continuous Dimming and high VT |  |
| 52 | Daylighting Tests Large OfficeCZ-7 WWR-60%,Stepped Dimming |  |
| 53 | Daylighting Tests Large OfficeCZ-7 WWR-60%,Stepped Dimming and high VT |  |
| 54 | Daylighting Tests Large OfficeCZ-1 Continuous Dimming | Large Office CZ-1 Baseline |
| 55 | Daylighting Tests Large OfficeCZ-1 Continuous Dimming and high VT |  |
| 56 | Daylighting Tests Large OfficeCZ-1 Stepped Dimming |  |
| 57 | Daylighting Tests Large OfficeCZ-1 Stepped Dimming and high VT |  |
| 58 | Daylighting Tests Large OfficeCZ-1 WWR-20%,Continuous Dimming | Large Office CZ-1 WWR-20% Baseline |
| 59 | Daylighting Tests Large OfficeCZ-1 WWR-20%,Continuous Dimming and high VT |  |
| 60 | Daylighting Tests Large OfficeCZ-1 WWR-20%,Stepped Dimming |  |
| 61 | Daylighting Tests Large OfficeCZ-1 WWR-20%,Stepped Dimming and high VT |  |
| 62 | Daylighting Tests Large OfficeCZ-1WWR-60%,Continuous Dimming | Large Office CZ-1 WWR-60% Baseline |
| 63 | Daylighting Tests Large OfficeCZ-1 WWR-60%,Continuous Dimming and high VT |  |
| 64 | Daylighting Tests Large OfficeCZ-1 WWR-60%,Stepped Dimming |  |
| 65 | Daylighting Tests Large OfficeCZ-1 WWR-60%,Stepped Dimming and high VT |  |
| 66 | Daylighting Tests Stand Alone Retail CZ-7 Skylight VT | Stand Alone Retail CZ-7 Baseline |
| 67 | Daylighting Tests Stand Alone Retail CZ-7 SRR-5% |  |
| 68 | Daylighting Tests Stand Alone Retail CZ-7 SRR-5% and high VT |  |
| 69 | Daylighting Tests Stand Alone Retail CZ-1 Skylight VT | Stand Alone Retail CZ-1 Baseline |
| 70 | Daylighting Tests Stand Alone Retail CZ-1 SRR-5% |  |
| 71 | Daylighting Tests Stand Alone Retail CZ-1 SRR-5% and high VT |  |
| 72 | Equipment test Medium OfficeCZ-15 low EPD | Medium Office CZ-15 Baseline |
| 73 | Equipment test Medium OfficeCZ-15 high EPD |  |
| 74 | Equipment test Medium OfficeCZ-16 low EPD | Medium Office CZ-16 Baseline |
| 75 | Equipment test Medium OfficeCZ-16 high EPD |  |
| 76 | Equipment test Medium OfficeCZ-6 low EPD | Medium Office CZ-6 Baseline |
| 77 | Equipment test Medium OfficeCZ-6 high EPD |  |
| 78 | Equipment test Fast food RestaurantCZ-6 high Process load | Fast food Restaurant CZ-6 Baseline |
| 79 | PTAC Test Strip Mall CZ-6 Low Eff | PTAC Strip Mall CZ-6 Baseline |
| 80 | PTAC Test Strip Mall CZ-6 High Eff |  |
| 81 | PTAC Test Strip Mall CZ-15 Low Eff | PTAC Strip Mall CZ-15 Baseline |
| 82 | PTAC Test Strip Mall CZ-15 High Eff |  |
| 83 | PTAC Test Strip Mall CZ-16 Low Eff | PTAC Strip Mall CZ-16 Baseline |
| 84 | PTAC Test Strip Mall CZ-16 High Eff |  |
| 85 | PTHP Test Strip Mall CZ-6 Low Eff | PTHP Strip Mall CZ-6 Baseline |
| 86 | PTHP Test Strip Mall CZ-6 High Eff |  |
| 87 | PTHP Test Strip Mall CZ-15 Low Eff | PTHP Strip Mall CZ-15 Baseline |
| 88 | PTHP Test Strip Mall CZ-15 High Eff |  |
| 89 | PTHP Test Strip Mall CZ-16 Low Eff | PTHP Strip Mall CZ-16 Baseline |
| 90 | PTHP Test Strip Mall CZ-16 High Eff |  |
| 91 | PSZ Test Strip Mall CZ-6 Low Eff | PSZ Strip Mall CZ-6 Baseline |
| 92 | PSZ Test Strip Mall CZ-6 High Eff |  |
| 93 | PSZ Test Strip Mall CZ-15 Low Eff | PSZ Strip Mall CZ-15 Baseline |
| 94 | PSZ Test Strip Mall CZ-15 High Eff |  |
| 95 | PSZ Test Strip Mall CZ-16 Low Eff | PSZ Strip Mall CZ-16 Baseline |
| 96 | PSZ Test Strip Mall CZ-16 High Eff |  |
| 97 | PVAV Test Medium Office CZ-6 Low Eff | PVAV Medium Office CZ-6 Baseline |
| 98 | PVAV Test Medium Office CZ-6 High Eff |  |
| 99 | PVAV Test Medium Office CZ-15 Low Eff | PVAV Medium Office CZ-15 Baseline |
| 100 | PVAV Test Medium Office CZ-15 High Eff |  |
| 101 | PVAV Test Medium Office CZ-16 Low Eff | PVAV Medium Office CZ-16 Baseline |
| 102 | PVAV Test Medium Office CZ-16 High Eff |  |
| 103 | VAV Test Large Office CZ-6 Low Eff | VAV Large Office CZ-6 Baseline |
| 104 | VAV Test Large Office CZ-6 Low Eff |  |
| 105 | VAV Test Large Office CZ-15 Low Eff | VAV Large Office CZ-15 Baseline |
| 106 | VAV Test LargeOffice CZ-15 Low Eff |  |
| 107 | VAV Test LargeOffice CZ-16 Low Eff | VAV Large Office CZ-16 Baseline |
| 108 | VAV Test LargeOffice CZ-16 Low Eff |  |
| 109 | FanCoil Test Strip Mall CZ-6 Low Eff | FanCoil Strip Mall CZ-6 Baseline |
| 110 | FanCoil Test Strip Mall CZ-6 High Eff |  |
| 111 | FanCoil Test Strip Mall CZ-15 Low Eff | FanCoil Strip Mall CZ-15 Baseline |
| 112 | FanCoil Test Strip Mall CZ-15 High Eff |  |
| 113 | FanCoil Test Strip Mall CZ-16 Low Eff | FanCoil Strip Mall CZ-16 Baseline |
| 114 | FanCoil Test Strip Mall CZ-16 High Eff |  |
| 115 | VRF Test Strip Mall CZ-6 Low Eff | PSZ Strip Mall CZ-6 Baseline |
| 116 | VRF Test Strip Mall CZ-6 High Eff |  |
| 117 | VRF Test Strip Mall CZ-15 Low Eff | PSZ Strip Mall CZ-15 Baseline |
| 118 | VRF Test Strip Mall CZ-15 High Eff |  |
| 119 | VRF Test Strip Mall CZ-16 Low Eff | PSZ Strip Mall CZ-16 Baseline |
| 120 | VRF Test Strip Mall CZ-16 High Eff |  |

## Results Documentation

The applicant shall perform simulations for all tests specified in Section VII and report results in the forms provided in Appendix A.